# 1. Goals and Objectives

The primary goals of the audit are to:

- Identify the causes and contributing factors associated with combined-product spillage within the specified areas of the product transport and processing system.

- Identify the locations and operating conditions under which spillage occurs.

- Determine whether individual spillage areas are associated with a single cause or with a combination of contributing factors.

- Define practical remedial actions, engineering controls, maintenance requirements, operating procedures, or policies that may be implemented to eliminate, reduce, or control the identified spillage.

# 2. Terminology

## 2.1 Combined Product

A mixture of lump ore and fines conveyed together as a single material stream.

## 2.2 Fines

Small or finely divided ore particles forming the finer fraction of the conveyed material. This may include very fine particulate material or ore dust.

## 2.3 Belt Tracking

The lateral alignment of the conveyor belt along its intended travel path.

Correct belt tracking occurs when the belt remains centred across the conveyor structure and does not exhibit excessive lateral drift toward either side.

- Correct Belt Tracking: The belt remains centred along its intended travel path.

- Belt Mistracking: The belt moves or runs laterally away from its intended centred position.

- Tracking Deviation: The measurable lateral displacement of the conveyor belt from its intended centreline.

### 2.4 Belt Carryback

Residual fines or product that remain adhered to the conveyor belt after the main product has been discharged and are carried onto the return side of the belt.

### 2.5 Belt Scraper / Belt Cleaner

A mechanical device positioned at or near a conveyor discharge point to remove material adhering to the belt and, where correctly configured, direct the removed carryback back into the intended product or discharge containment path.

### 2.6 Wet Fines / Slurry

Fines containing sufficient moisture or process water to behave as wet particulate material or a slurry-like mixture. Wet fines may be transported with the product or may result from the screening and washing process.

2.7 Conveyor Side Skirt / Side-Containment Skirting System

A material-containment arrangement installed along the side of a conveyor belt to help retain conveyed product within the usable carrying width. For the sample conveyors, the current working concept consists of a fixed side support with flexible rubber skirting positioned close to, but not normally in contact with, the belt. Final geometry, clearance and construction remain subject to engineering verification.

# 3. Report Philosophy and Assessment Approach

The purpose of the report is to document and communicate the findings of an audit conducted across selected sections of the combined-product handling and processing plant.

The audit is primarily intended to establish:

- where spillage occurs;

- what type of spillage is present;

- the mechanism or condition causing the spillage;

- whether the condition results from a single cause or a combination of contributing factors; and

- what practical actions can be taken to eliminate, reduce, contain, or otherwise control the spillage.

The report is not intended to be a fully quantified technical study of total product losses. For example, it is not necessary at this stage to determine the exact mass or volume of material spilled from every conveyor, transfer point, or processing subsystem.

The assessment therefore places greater emphasis on identifying spillage mechanisms and contributing conditions than on calculating the absolute quantity of material lost.

The extent of visible spillage may, however, be used as an indicator of the severity or persistence of a problem. For example, extensive accumulation within a particular area may indicate:

- a recurring operating condition;

- inadequate containment or cleaning;

- ineffective maintenance;

- a significant individual defect; or

- several contributing factors acting together.

The observed extent and distribution of the spillage should therefore assist in determining the probable causes and in prioritising suitable remedial actions.

Remedial actions may include, depending on the condition identified:

- mechanical modifications;

- conveyor alignment or tracking correction;

- improved product containment;

- improved fines management;

- maintenance interventions;

- structural repairs or modifications;

- improved housekeeping and spillage-removal practices;

- operational changes;

- procedures or work instructions; and

- preventative or monitoring measures.

# 4. Inspection Scope

The audit included selected sections of both the surface and underground combined-product handling and processing systems.

## 4.1 Surface Processing Plant

### 4.1.1 Screening Processing Section

- W10 Conveyor — Primary Feed

- CV01, CV02, CV03, CV04, CV06, CV09, CV14 and CV15

- Screen / screen-chute assemblies

Scope note: CV05, CV07, CV08, CV10, CV11, CV12 and CV13 are excluded from integration into the report. CV15 and the screen / screen-chute assemblies were added to the inspection scope during Questionnaire V1.

### 4.1.2 Loading Station Processing Section

- L1 Conveyor — Primary Feed

- L2 Conveyor

- L3 Conveyor

- L4 Conveyor

- Sample Conveyor 1

- Sample Conveyor 2

- Sample Conveyor 3

## 4.2 Underground Sections

### 4.2.1 Central Block

- Crusher

- UC05

- UC04

- UC03

- UC02

- UC01

Operational grouping note: UC05, UC04, UC03, UC02 and UC01 form one logical continuous product-transport system divided into physical conveyor sections. The completed Questionnaire V1 therefore records the observations for these sections as a combined system.

### 4.2.2 Secondary Plant

- U3 Conveyor

- U4 Conveyor — Tripper Car

### 4.2.3 Silos / Feeders

- U5A Conveyor

- U5B Conveyor

# 5. General Spillage Categories

The spillage identified during the audit can initially be grouped into three broad categories:

- Water-related spillage or leakage

- Fines spillage

- Solid-product / combined-product spillage

Although these categories are considered separately below, they may interact. For example, water leakage may transport fines into surrounding areas, while poor belt tracking may result in both lump ore and fines being discharged from the side of a conveyor.

# 6. Water-Related Spillage

## 6.1 Process Context

Water-related spillage occurs primarily within the screening plant, where water is introduced into the combined-product stream as part of the washing and screening process.

The water assists in separating the fines from the larger ore fraction.

## 6.2 Spillage Mechanism

Spillage occurs where the process water is not fully contained within the chute, screening, or associated process equipment.

The water may consequently leak, splash, overflow, or escape from the intended process path into the surrounding plant area.

Depending on the location and operating condition, the escaping water may also carry fines or other particulate material with it.

## 6.3 Primary Location

This form of spillage appears to occur primarily within the Screening Processing Section.

## 6.4 Potential Consequences

Water escaping from the process may contribute to:

- wet or slippery walking surfaces;

- transport of fines into surrounding areas;

- accumulation of wet fines or slurry;

- accelerated corrosion or degradation of surrounding equipment and structures;

- increased housekeeping requirements; and

- reduced effectiveness of general spillage-control measures.

# 7. Fines Spillage

## 7.1 Generation and Process Context

Fines are present throughout much of the combined-product handling process.

A significant proportion of the fines may be generated during crushing and subsequent material handling.

The fines are then transported together with the larger ore fraction through the conveyor system until the washing and screening process separates the finer material from the larger product.

## 7.2 Primary Fines-Spillage Mechanism — Carryback

A major fines-spillage mechanism occurs when fines remain adhered to the conveyor belt after the main product has been discharged at a transfer or discharge point.

Instead of leaving the belt with the bulk product, a portion of the fines remains attached to the belt surface and travels onto the return side of the conveyor.

This condition may be referred to as belt carryback.

As the return belt travels back through the conveyor system, the adhered fines may progressively detach from the belt.

The fines can then:

- fall from the underside of the return belt;

- be deposited below the conveyor;

- accumulate around conveyor structures;

- fall into walkways or access areas; or

- be dislodged when the belt comes into contact with mechanical components such as return rollers and other conveyor-support components.

## 7.3 Distribution

Because carryback occurs on the return side of the conveyor, fines spillage can potentially extend for a considerable distance downstream of the original discharge point.

This means the physical location where fines accumulate is not necessarily the location where the underlying cause originates.

Fines spillage may therefore potentially occur throughout significant portions of the conveyor system.

## 7.4 When Fines Become a Spillage Problem

Fines become problematic where they accumulate outside the intended product-handling path, particularly where they:

- accumulate within walkways or access areas;

- collect beneath conveyors;

- build up around rollers or other conveyor components;

- interfere with equipment operation or maintenance access;

- create excessive housekeeping requirements;

- become wet and form mud or slurry;

- migrate into surrounding equipment or structures; or

- create an unsafe working environment.

# 8. Solid Product / Combined-Product Spillage

Solid-product spillage refers primarily to lump ore or the combined product physically leaving the intended conveyor or transfer path.

Several different mechanisms may result in this form of spillage.

## 8.1 Belt Mistracking

When conveyor belt mistracking causes the belt to move laterally away from its intended centred position, the effective conveying area becomes displaced relative to the intended product path.

This may create an exposed area or gap along one side of the conveyor, allowing ore or combined product to spill from the side of the belt.

The severity of the resulting spillage may depend on factors such as:

- the magnitude of the tracking deviation;

- the quantity of material being conveyed;

- material loading distribution;

- conveyor speed;

- belt trough profile;

- containment arrangements; and

- the position of the mistracking relative to transfer points or other equipment.

## 8.2 Conveyor Trips and Restart Preparation

When a loaded conveyor trips, it may be necessary to remove a portion of the conveyed product from the belt in order to reduce the static load and facilitate conveyor restart.

If the removed material is deposited adjacent to the conveyor or within surrounding plant areas without an effective recovery or containment method, this material becomes a further source of product spillage.

The resulting spillage is therefore not necessarily caused by a conveyor defect alone, but may arise as a secondary consequence of the trip and subsequent restart procedure.

## 8.3 Transfer and Containment Losses

Solid product may also escape where the material is transferred between conveyor belts or through chutes if the product trajectory is not adequately controlled or contained.

Potential contributing factors may include:

- inadequate chute containment;

- worn or damaged chute components;

- incorrect material trajectory;

- excessive material loading;

- belt mistracking at the transfer point;

- blockages or restrictions;

- poor skirt or sealing arrangements; or

- structural or mechanical deterioration.

These mechanisms should be assessed individually at each identified spillage location.

## 8.4 When Solid-Product Spillage Becomes a Problem

Solid-product spillage becomes particularly significant when material:

- enters walkways or access routes;

- accumulates around operating equipment;

- interferes with plant operation or maintenance;

- obstructs access to equipment;

- falls from elevated conveyors;

- creates additional manual handling requirements; or

- creates the potential for injury or damage to personnel, structures, or equipment below.

Material falling from an elevated conveyor may represent a greater consequence than equivalent spillage occurring at ground level because of the additional impact energy associated with the falling product.

## 8.5 Potential Locations

Solid-product spillage may occur throughout the conveyor system, depending on the particular mechanical, operational, transfer, or containment condition present at each location.

# 9. Relationship Between Spillage Location and Root Cause

An important principle for the audit is that the location where spilled material is found may not necessarily be the location where the original problem occurs.

Examples include:

- fines adhering to a belt at a discharge point and only falling from the belt further along the return path;

- belt mistracking developing upstream and causing product loss at a downstream containment point;

- product removed following a conveyor trip accumulating some distance from the point that caused the trip; and

- water carrying fines away from the original leakage point.

The investigation should therefore distinguish between:

- the location where the spillage is observed;

- the mechanism by which the material reached that location; and

- the original condition or root cause responsible for the spillage.

This distinction will be important when determining suitable corrective actions.

# 10. Initial Problem Areas / Restrictions Identified

The audit has identified several broader problem areas that may influence the occurrence, severity, or management of spillage.

These should be developed further as the individual findings are assessed.

## 10.1 Conveyor Mechanical Component Maintenance

The condition, adjustment, wear, and maintenance of conveyor components may influence:

- belt tracking;

- belt cleaning;

- material containment;

- product transfer;

- carryback; and

- general conveyor reliability.

Relevant components may include rollers, idlers, pulleys, belt cleaners, skirting, chute components, tensioning systems, and associated alignment components.

## 10.2 Structural Maintenance

Structural deterioration, deformation, movement, or misalignment may affect conveyor geometry and the performance of associated containment systems.

Structural maintenance may therefore have a direct or indirect influence on spillage.

## 10.3 Confined-Space and Restricted-Access Areas

Some spillage locations may be difficult to inspect, clean, maintain, or modify because of confined-space requirements or restricted physical access.

These restrictions may influence both:

- the persistence of existing spillage; and

- the practicality of implementing corrective actions.

## 10.4 Fines Control

Fines require specific consideration because their behaviour differs from that of larger lump ore.

Control measures may need to address:

- carryback;

- belt cleaning;

- dust;

- wet fines or slurry;

- accumulation beneath conveyors; and

- migration into surrounding plant areas.

## 10.5 Spillage Handling

The method by which spilled product is collected, removed, stored, and returned to the process should be assessed.

Spillage handling should not only address the removal of material after an event but should also consider how recovery activities themselves may contribute to secondary spillage.

## 10.6 Spillage-Control Measures

Existing preventative and containment measures should be assessed to determine whether they are:

- present;

- correctly installed;

- appropriate for the application;

- maintained;

- effective under normal operating conditions; and

- effective during abnormal operating conditions such as conveyor trips or blockages.

# 11. Initial Cause-and-Effect Framework

The individual observations within the report can be assessed using the following general sequence:

Observed Spillage → Spillage Type → Immediate Mechanism → Contributing Condition(s) → Root Cause → Consequence → Existing Control → Recommended Remedial Action

For example:

Product accumulation below conveyor
→ Fines
→ Carryback from return belt
→ Fines adhering to belt after discharge
→ Ineffective or inadequate belt cleaning
→ Fines deposited along conveyor return path
→ Existing cleaner / housekeeping practices assessed
→ Improve belt-cleaning effectiveness and verify cleaner configuration.

Or:

Lump ore adjacent to conveyor
→ Solid-product spillage
→ Product leaves side of belt
→ Belt operating laterally offset
→ Belt mistracking / conveyor alignment issue
→ Product loss and potential personnel/equipment hazard
→ Existing tracking arrangements assessed
→ Correct underlying tracking condition and verify containment.

# 12. Intended Outcome of the Audit

The final report should ultimately provide a practical understanding of:

- Where is the spillage occurring?

- What type of material is being spilled?

- How is the material escaping from the process?

- Why is it occurring?

- Is the problem isolated or recurring?

- Is one cause responsible, or are several factors interacting?

- What consequence does the condition create?

- What measures are currently in place?

- Why are those measures not fully effective?

- What corrective or preventative actions should be implemented?

The primary objective is therefore not simply to document spilled material, but to establish the relationship between the observed spillage, the operating or mechanical mechanism responsible for it, and the practical action required to control or eliminate the problem.

# 13. Questionnaire V1 Findings - Current Inspection Baseline

Questionnaire V1 has established the first asset-by-asset baseline of where spillage was observed, what general form the spillage took, its qualitative extent, and the supporting photographic or video references. The following information is folded into these contextual notes as source material for later report development. It should not yet be treated as the final root-cause analysis or final recommendation set.

## 13.1 Surface Processing Plant - Screening Processing Section

The dominant pattern in the screening section is fines-related spillage, with both dry and wet fines present depending on the process location. Carryback on the return side of the conveyors is a recurring mechanism. The screening process also introduces a separate water-containment problem at the screens and screen chutes.

### 13.1.1 W10 Conveyor - Primary Feed

- Spillage was recorded. The observed material included dry fines, carryback, lump ore / solid product and mixed combined product. Batch D visual review directly confirmed substantial dry lump/mixed-product and fines accumulation within the enclosed tunnel sections.

- The spillage was observed at the feed area, along the carrying and return sides, beneath the conveyor, around conveyor components and in access areas. The visible accumulation was assessed as localised to moderate rather than widespread. Individual photographs show locally severe piles within the inspected tunnel sections, but this does not by itself overturn the asset-level localised-to-moderate classification.

- W10 is the primary surface-plant feed for product arriving from underground. It runs in an enclosed tunnel and is therefore associated with confined-space / restricted-access considerations.

- Evidence references: W10-REF-001 through W10-REF-012.

### 13.1.2 CV01

- Wet fines, dry fines and return-side carryback were recorded. Solid-product and mixed-product spillage were not recorded in Questionnaire V1. Batch D photographs directly confirm large wet/damp fines-slurry accumulations and standing water/slurry below the conveyor.

- A damaged / seized return support roller was identified as a mechanical-maintenance issue. CV01-REF-001 directly shows severe visible surface damage/abrasion on the identified roller; however, a static photograph cannot independently prove the recorded seized operating condition.

- Evidence references: CV01-REF-001 through CV01-REF-004; damaged / seized roller specifically referenced in CV01-REF-001.

### 13.1.3 CV02

- Dry fines, carryback, lump ore / solid product and mixed combined-product spillage were recorded. Batch B photographs directly confirm dry fines together with mixed-size/lump-product accumulation around the transfer and return-side support areas.

- The visible condition was assessed as localised to moderate accumulation.

- Evidence references: CV02-REF-001 through CV02-REF-004.

### 13.1.4 CV03

- No spillage was recorded during Questionnaire V1. The remaining spillage classifications were marked not applicable for the inspected condition. CV03-REF-002 and CV03-REF-003 visually support this clean/no-significant-spillage inspected-condition baseline.

- Evidence references: CV03-REF-001 through CV03-REF-003.

### 13.1.5 CV04

- Wet fines, dry fines, carryback and mixed combined-product spillage were recorded. Batch D photographs directly show saturated wet-fines/slurry deposits, including visible droplets/wet fallout at CV04-REF-002, together with substantial accumulated fines below the conveyor.

- The visible accumulation was assessed as extensive / widespread.

- Evidence references: CV04-REF-001 through CV04-REF-005.

### 13.1.6 CV06

- Dry fines and return-side carryback were recorded, with trace / minor deposits observed.

- Significant belt-cleaner maintenance deficiencies were documented: CV06-REF-004 shows the scraper support bushing displaced from its housing, and CV06-REF-005 shows the head scraper blades completely missing.

- A broader review of the scraper support / adjustment arrangement was proposed in the questionnaire notes.

- Evidence references: CV06-REF-001 through CV06-REF-005.

### 13.1.7 CV09

- Wet fines / slurry and return-side carryback were recorded, with extensive / widespread accumulation.

- The existing scraper arrangement was identified as ineffective. The scraper is positioned outside the discharge chute, which allows removed wet carryback to fall into the surrounding environment instead of being returned to the intended discharge path.

- Residual carryback remaining on the belt is subsequently removed by downstream support rollers, spreading fines to other areas including lower screen-house floors.

- A working design concept was recorded: relocate / enclose the scraper within the discharge-chute arrangement, provide positive spring-loaded contact, and include wear-limit stops to prevent the scraper system from over-travelling toward the belt once the wear elements reach their allowable limit.

- Evidence references: CV09-REF-001 through CV09-REF-005 and videos CV09-REF-006 through CV09-REF-008; scraper condition / arrangement specifically referenced in CV09-REF-003 and CV09-REF-008.

### 13.1.8 CV14

- Dry fines and return-side carryback were recorded. The visible accumulation was mainly trace to localised. Batch D photographs support this classification, showing small access-area deposits and a localised fines/carryback mound beneath the return-side structure.

- Evidence references: CV14-REF-001 and CV14-REF-002.

### 13.1.9 CV15

- CV15 was added to the inspection scope during Questionnaire V1.

- Wet fines, dry fines and carryback were recorded, with extensive / widespread accumulation. Batch D photographs strongly verify extensive distributed fines/carryback accumulation on grating, cross-bracing and structural members around the CV15 discharge area. The supplied still photographs primarily show dry or dried deposits; the wet-fines component remains a source-recorded inspection observation rather than a condition independently obvious from this still set.

- The noted origin / reference area is around chute E-3181. CV15-REF-003 directly confirms the E-3181 chute identification.

- Evidence references: CV15-REF-001 through CV15-REF-008.

### 13.1.10 Screens and Screen Chutes

- The screening process uses water to wash the combined product and separate fines from the larger ore fraction. Batch C visual review directly verified process-water spray/runoff escaping from / around the operating screen/chute containment rather than remaining fully contained within the process equipment (particularly videos Screen-REF-016 and Screen-REF-017). The exact equipment-level escape mechanism remains unresolved.

- The resulting water and wet-fines exposure is associated with long-term corrosion of screen support structures, internal screen-building floor supports and surrounding areas. Batch C photographs directly show severe corrosion and deterioration at multiple screen/deck-support interfaces.

- Support members identified during the inspection/contextual review as load-bearing were observed to be damaged / cut. Batch C photographs directly verify multiple physical cut-outs/notches and severe deterioration in these support members. The reason for the cuts has not been confirmed. The working hypothesis that supports may have been cut to assist screen replacement / installation remains an unverified inspector hypothesis.

- The damaged load-bearing supports require assessment and rectification by an appropriately qualified structural authority. If replacement geometry is changed to provide future installation clearance, the required dimensions and revised support positions must be quantified and structurally verified.

- Evidence references: Screen-REF-001 through Screen-REF-014 and videos Screen-REF-015 through Screen-REF-017.

## 13.2 Surface Processing Plant - Loading Station Processing Section

The loading-station section shows a stronger solid-product / mixed-product spillage component than most of the other inspected areas. Dry fines remain a common condition, while the sample conveyors introduce a separate loading-capacity / containment problem.

### 13.2.1 L1 Conveyor - Primary Feed

- Dry fines, carryback, lump ore / solid product and mixed combined-product spillage were recorded, with extensive / widespread accumulation.

- Structural damage associated with the belt / conveyor area is directly supported by LoadoutStationL1-REF-007, which shows smooth wear slots/notches in angular supports immediately adjacent to the running belt path. In the captured operating image, the belt is clear of the supports, supporting previous/intermittent rather than continuous contact.

- Tunnel structural damage is directly recorded in LoadoutStationL1-REF-003 through LoadoutStationL1-REF-006, including severe concrete spalling/deterioration and exposed reinforcement/corrosion staining.

- Severe steel corrosion and local section loss/perforation are directly recorded in LoadoutStationL1-REF-008 and LoadoutStationL1-REF-009.

- Evidence set: LoadoutStationL1-REF-001 through LoadoutStationL1-REF-018.

### 13.2.2 L2 Conveyor

- No significant spillage was recorded during Questionnaire V1. Batch E visual review supports this inspected-condition baseline: the supplied photographs and the approximately 28-second LoadoutStationL2-REF-005.mp4 sequence show comparatively clean access, transfer and guarded-equipment areas with no significant active material loss or large existing spillage deposit visible in the captured condition.

- Evidence references: LoadoutStationL2-REF-001 through LoadoutStationL2-REF-006, including the separate video file LoadoutStationL2-REF-005.mp4. Because REF-005 exists as both a .jpg and .mp4, the media type / exact filename must be stated when cited.

### 13.2.3 L3 Conveyor

- Dry fines and carryback were recorded. Solid-product / mixed-product spillage was not recorded in Questionnaire V1.

- A tail-end pulley / belt offset was observed and recorded in LoadoutStationL3-REF-001. The relationship between this offset and any current spillage mechanism should be addressed in the next root-cause iteration rather than assumed at this stage.

- Evidence references: LoadoutStationL3-REF-001 through LoadoutStationL3-REF-008.

### 13.2.4 L4 Conveyor

- Dry fines and lump ore / solid-product spillage were recorded. Batch E photographs directly confirm localised coarse-product and fines accumulation in lower channels / troughs and on support members immediately adjacent to the conveyor, particularly in LoadoutStationL4-REF-003 and LoadoutStationL4-REF-004. The main access walkway remains comparatively clean in the wider views, and the existing trace-to-localised asset-level classification remains appropriate.

- Operating views LoadoutStationL4-REF-007 and LoadoutStationL4-REF-008 show coarse product being conveyed generally within the belt trough, with residual material on adjacent structure. No active material-fall event is captured in the still photographs; the images therefore verify the accumulated local spillage condition rather than the exact instantaneous loss mechanism. Evidence references: LoadoutStationL4-REF-001 through LoadoutStationL4-REF-008.

### 13.2.5 Sample Conveyor 1

- Wet fines, dry fines, carryback, lump ore / solid product and mixed combined-product spillage were recorded, with extensive / widespread accumulation. Batch E visual review strongly confirms widespread dry / dried fines and coarse-product accumulation on floors, stairs, access grating, platforms and structural members at multiple elevations.

- LoadoutStationSample1-REF-005, -006 and -007 show thick adhered / dried deposits on structural steel, including pronounced run / drip-like morphology. These features visually support a history of sticky or moisture-laden material deposition, but the still photographs do not independently prove that the material was actively wet at the instant photographed or identify the water source. The wet-fines classification therefore remains source-recorded inspection information, while the physical adhered deposits are directly verified.

Key direct evidence includes LoadoutStationSample1-REF-008 (coarse product across access grating), LoadoutStationSample1-REF-010 (large mixed accumulation within the lower / adjacent structure) and LoadoutStationSample1-REF-011 (major coarse / lump-product accumulation retained on and within surrounding structure). The photographs prove distributed accumulation but do not independently establish that every deposit originated specifically from carryback. Overall evidence set: LoadoutStationSample1-REF-001 through LoadoutStationSample1-REF-011.

### 13.2.6 Sample Conveyor 2

- Dry fines, carryback, lump ore / solid product and mixed combined-product spillage were recorded. Batch B photographs directly confirm dry fines together with mixed-size/lump-product accumulation around the transfer and return-side support areas.

- The questionnaire identifies an adverse relationship between the size / volume of product loaded and the available belt size / containment capacity. Product feed onto the sample belt should therefore be investigated and, if required, throttled / restricted so that the belt is not loaded beyond its practical containment capability.

- A working containment concept was proposed: fit side containment rails / sideboards along the conveyor to help retain product on the belt. The final engineering term, design and permissible contact / clearance arrangement still require definition.

- Specific evidence references for the loading / containment issue: LoadoutStationSample2-REF-004 through LoadoutStationSample2-REF-007. Overall evidence set: LoadoutStationSample2-REF-001 through LoadoutStationSample2-REF-008.

### 13.2.7 Sample Conveyor 3

- Lump ore / solid-product and mixed combined-product spillage were recorded. Product clearing following a conveyor trip / restart event was also marked as applicable.

- The same product-loading / belt-capacity concern recorded for Sample Conveyor 2 was noted for Sample Conveyor 3.

- Belt mistracking was additionally observed on Sample Conveyor 3.

- A similar side containment rail / sideboard concept was proposed as a working measure, subject to proper engineering definition and verification.

- Specific evidence references for the loading / containment issue: LoadoutStationSample3-REF-002 through LoadoutStationSample3-REF-007. Overall evidence set: LoadoutStationSample3-REF-001 through LoadoutStationSample3-REF-013.

## 13.3 Underground - Central Block

### 13.3.1 Crusher

- Dry fines, carryback and lump ore / solid-product spillage were recorded. Batch F visual review directly confirms mixed coarse/lump material and fines accumulated around and beneath the crusher/conveyor area.

- The visible accumulation remains appropriately assessed as trace to moderate at asset level. Individual locations are locally significant, including UGCrusher-REF-001 and -002, but the photographic set does not show uniformly widespread accumulation through the full installation. UGCrusher-REF-001 also shows standing water/puddling around the accumulated product; the water source is not established by the photograph.

- Fine material located beneath/adjacent to the belt and return-side structure is consistent with the recorded carryback mechanism, but the still photographs do not independently prove that every deposit originated specifically from carryback. Evidence references: UGCrusher-REF-001 through UGCrusher-REF-005.

### 13.3.2 UC05 / UC04 / UC03 / UC02 / UC01 Continuous Conveyor System

- The five conveyor sections form one logical continuous product-transport system from the crusher to the transfer toward the Secondary Plant / U3 Conveyor. Questionnaire V1 therefore combines the observations for these sections.

- Dry fines, carryback and mixed combined-product spillage were recorded, with moderate accumulation distributed along the conveyor route. Batch F visual review strongly verifies repeated fines accumulation beneath and alongside the conveyor at multiple locations, with mixed coarse fragments present in some areas. Some individual local mounds are large, while other route sections are comparatively clear; the overall moderate, distributed classification remains appropriate.

- The area between UC05 and UC03 had reportedly been cleared shortly before the inspection. UGUC05_04_03_02_01-REF-007 directly shows a comparatively clear inspected condition, but the photograph alone cannot establish when or why the area was cleaned. The recent-cleaning explanation therefore remains source-recorded inspection information. The same evidence set directly shows substantial fines accumulation elsewhere in the continuous route, including REF-006, -008 and -015/-016.

- The location of fines directly beneath/adjacent to the belt and support structure is consistent with carryback redistribution, but the static photographs do not independently prove carryback as the sole origin of every deposit. Evidence references: UGUC05_04_03_02_01-REF-001 through UGUC05_04_03_02_01-REF-016.

## 13.4 Underground - Secondary Plant

### 13.4.1 U3 Conveyor

- Dry fines, carryback and mixed combined-product spillage were recorded. Batch F visual review directly confirms local fines/mixed-material accumulation immediately beneath and adjacent to the conveyor structure, particularly in UGU3-REF-004, -005 and -007. The trip / restart clearing condition remains Unknown for this conveyor.

- The visible accumulation remains appropriately assessed as localised to moderate. UGU3-REF-002, -003, -006 and -008 also show coarse broken rock/material along the underground roadway adjacent to the conveyor; because this is an excavated underground roadway, the photographs alone cannot establish that every coarse fragment originated from conveyor spillage. Strongest direct report evidence should therefore favour deposits physically beneath/adjacent to the conveyor.

- Evidence references: UGU3-REF-001 through UGU3-REF-008.

### 13.4.2 U4 Conveyor — Tripper Car

- Dry fines, carryback, lump ore / solid product and mixed combined-product spillage were recorded, with extensive / widespread accumulation.

- The tail-end pulley was recorded by the inspector as misaligned. The associated belt mistracking is visually evident in the referenced evidence and product loss/accumulation is concentrated along one side of the conveyor area.

- The current working cause sequence is therefore: source-recorded tail-end pulley misalignment -> visually verified belt mistracking -> lateral product loss from the belt. The supplied photographs/video support the alignment concern but do not provide a fixed survey reference or dimensional measurement sufficient to independently quantify the pulley misalignment relative to the tripper structure.

- Evidence references: UGU4-REF-001 through UGU4-REF-009 and video UGU4-REF-010; pulley misalignment specifically referenced in UGU4-REF-008 / UGU4-REF-010, mistracking in UGU4-REF-004 / UGU4-REF-009, and product loss in UGU4-REF-007. Questionnaire V2 additionally identifies UGU4-REF-011 and UGU4-REF-012 as references showing the different/custom U4 cleaner arrangement.

## 13.5 Underground - Silos / Feeders

### 13.5.1 U5A Conveyor

- Dry fines and return-side carryback were recorded, with extensive / widespread accumulation. Batch F visual review strongly confirms large fines mounds beneath and alongside the conveyor at multiple locations, including long longitudinal accumulations in the confined conveyor-side route.

- UG5A-REF-004 shows fines accumulated immediately beneath the return-belt / roller region, strongly consistent with return-side carryback geometry. The still photograph does not establish carryback as the sole source of all surrounding fines. The existing extensive / widespread classification is fully supported. Evidence references: UG5A-REF-001 through UG5A-REF-009.

### 13.5.2 U5B Conveyor

- Dry fines and return-side carryback were recorded. Batch F visual review confirms fines accumulation at several localised points around the conveyor and return-side structure. Some individual local deposits are sizeable, particularly UG5B-REF-001 and UG5B-REF-005, but the photographic set does not show the same continuous / widespread longitudinal accumulation seen at U5A. The existing trace-to-localised asset-level classification is therefore retained.

- UG5B-REF-003 and -004 show fines immediately beneath the return-belt / support region, strongly consistent with the recorded carryback mechanism; as elsewhere, static photographs do not prove carryback as the sole origin of every deposit. Evidence references: UG5B-REF-001 through UG5B-REF-006.

# 14. Cross-System Themes Emerging from Questionnaire V1

## 14.1 Fines and Carryback are a System-Wide Issue

The completed questionnaire indicates that dry fines and belt carryback are not isolated to a single plant area. They occur across the underground and surface conveyor transport system. The current working interpretation is that fines are not consistently removed and returned to the intended product path at conveyor discharge points, allowing material to remain on the return belt and be progressively redistributed beneath conveyors, around mechanical components and into surrounding plant areas.

## 14.2 Belt-Cleaner / Scraper Effectiveness

A recurring control weakness is emerging around belt cleaners / scrapers. Questionnaire V1 provides examples of both maintenance failure and possible design inadequacy. CV06 records displaced scraper-support hardware and missing blades, while CV09 records an arrangement that is both ineffective at removing all carryback and positioned outside the discharge-chute containment, allowing even the material that is removed to become spillage.

## 14.3 Product Loading and Containment

The sample conveyors indicate a separate mechanism in which the quantity and/or size of the product presented to the belt may exceed the belt section's practical ability to contain the load. This requires investigation of feed-rate / throughput control as well as physical side containment. Sample Conveyor 3 adds belt mistracking as a further contributing factor.

## 14.4 Belt Alignment and Tracking

Belt alignment and tracking remain important contributors where solid product leaves the belt. The strongest current evidence is at the U4 Tripper Car, where tail-end pulley misalignment was recorded during the inspection and belt mistracking plus one-side product loss/accumulation are visually documented. A tail-end offset is also recorded at L3 and mistracking is recorded at Sample Conveyor 3, but the complete causal chain for those locations still requires confirmation.

## 14.5 Water Containment and Corrosion

Water-related spillage is concentrated in the screening plant, where process water used to wash and separate fines is not fully contained within the screen / chute system. This creates wet-fines / slurry spillage and contributes to corrosion of structural members and surrounding plant areas. CV09 then transports wet fines downstream of the screening process and exhibits substantial wet carryback.

## 14.6 Structural Condition

Structural issues require a parallel workstream because they may be consequences of long-term spillage or independent defects that affect safe operation and maintainability. The current evidence includes corrosion and structural damage around the L1 tunnel / conveyor area and damaged / cut load-bearing screen supports. The screen-support condition requires formal structural assessment rather than assumption-based repair.

## 14.7 Operationally Generated Spillage

Spillage associated with conveyor trips / restart preparation should be treated differently from continuous mechanical leakage or carryback. It occurs episodically when product must be physically removed from a loaded belt to reduce restart load. The material then requires secondary handling and reintroduction into the process. Questionnaire V1 does not identify one fixed location for this condition because it depends on the particular trip, blockage or restart event.

## 14.8 Confined / Restricted Access

W10 is specifically identified as an enclosed tunnel / confined-space area. Restricted access can influence inspection, maintenance, cleaning and recovery of spilled material, and should therefore be considered when practical corrective actions are developed.

# 15. Working Remedial Concepts Captured from Questionnaire V1

The following items were the initial working concepts captured after Questionnaire V1. They remain part of the development history, but several have now been refined by Questionnaire V2. The current consolidated working remedial concepts are recorded in Section 20.

- Belt-cleaner / scraper improvement: review scraper design and maintenance condition across the conveyor system, with particular focus on CV06 and CV09.

- CV09 discharge / scraper concept: contain the scraper within an extended or revised discharge-chute arrangement so removed carryback is returned to the discharge stream; use a positive spring-loaded contact arrangement with suitable wear / travel limiters.

- Sample Conveyor 2 and 3 feed control: evaluate the upstream mechanism controlling sample-product throughput and restrict / meter the incoming product so the belt is not overloaded relative to its containment capability.

- Sample Conveyor 2 and 3 side containment: investigate side containment rails / sideboards as a secondary retention measure. The final geometry and terminology still require engineering definition.

- U4 Tripper Car: correct the tail-end pulley alignment and verify belt tracking through the full tripper-car operating range.

- Screen / chute water containment: identify the specific leakage / escape paths and modify containment so process water and wet fines remain within the intended process system.

- Screen support structure: obtain a qualified structural assessment and define repair / replacement requirements for damaged / cut load-bearing members, including any geometry changes needed for future screen installation clearance.

- L1 structural condition: evaluate the documented structural damage and corrosion to determine required repair and whether any of the condition is being accelerated by spillage or belt interaction.

# 16. Questionnaire V1 Open Items - Status After Questionnaire V2

Questionnaire V2 addressed the mechanism-focused questions generated from Questionnaire V1. The original V1 open-item list is therefore no longer current as a questionnaire task list. Its status is summarised below so that the development history remains traceable.

• Carryback / scraper deficiency - SUBSTANTIALLY RESOLVED AT CONCEPT LEVEL. V2 confirms a recurring carryback mechanism and records both design-related and maintenance-related cleaner deficiencies. Direct examples are CV06 and CV09. Cleaner presence and exact configuration were not visually verified at every conveyor, so system-wide design similarity remains an engineering inference requiring verification before standardisation.

• Common cleaner standard - PARTIALLY RESOLVED. The mine-wide existing standard is unknown. V2 develops a proposed common self-tensioning cleaner concept, with CV09 identified as the preferred pilot location. Wider rollout is conditional on successful testing and verification of physical applicability at each discharge point.

• CV09 cleaner / chute arrangement - RESOLVED TO WORKING CONCEPT. V2 confirms that the cleaner is outside the effective containment envelope and that successfully removed wet carryback can fall into the surrounding environment. Space appears available to revise/extend the chute, retain maintenance access, use self-tensioning contact and incorporate wear/travel limits. A secondary cleaner may also be required. Detailed dimensions remain open.

• Sample Conveyors 2 and 3 feed control - PARTIALLY RESOLVED. Excessive product quantity and product size relative to the narrow belt are confirmed concerns. The exact upstream control mechanism remains unclear and requires inspection/measurement before a specific throttling or metering design can be selected.

• Sample Conveyors 2 and 3 side containment - RESOLVED TO CONCEPT LEVEL. The working concept is a side-support / rubber-skirt containment system extending for a substantial length, potentially the full conveyor. Questionnaire V2 records approximately 250 mm height and 5-10 mm belt clearance as preliminary concepts only; final dimensions require engineering verification.

• L3 tail-end offset / mistracking - PARTIALLY RESOLVED. V2 records active, significant lateral offset, but the mechanical cause remains unknown. V1 did not record solid-product spillage at L3, and one V2 evidence entry incorrectly referenced the Sample Conveyor 3 photograph series. The L3-to-spillage causal relationship must therefore remain qualified pending verification.

• U4 Tripper Car - PARTIALLY RESOLVED. V2 indicates that the tail-pulley misalignment is fixed relative to the tripper-car structure rather than changing with car travel. The underlying mechanical source and exact adjustment details remain unknown. Post-correction tracking should be verified through the full travel range under operating conditions.

• Screen water escape - PARTIALLY RESOLVED. Water escape during normal operation and the general escape area are directly observed, but the equipment-level cause could not be identified safely while the screen was operating. A closer inspection during a suitable shutdown remains required.

• Cut / damaged screen supports - OPEN FOR STRUCTURAL ACTION. The load-bearing members are confirmed damaged/cut, but the reason remains unconfirmed. No completed structural assessment or scheduled repair was confirmed in V2. Any revised support geometry requires structural calculation and approval.

• L1 structural damage / corrosion - PARTIALLY RESOLVED. Evidence supports intermittent belt-to-structure contact associated with mistracking and a separate corrosion mechanism associated with fines/moisture accumulation. Formal structural assessment and repair status remain unknown.

• Housekeeping / spillage recovery - PARTIALLY RESOLVED. Manual shovelling, wheelbarrows and water wash-down are used in some areas, and clearing appears largely reactive/as-required. Exact frequencies, formal standards and the confirmed route for reintroducing recovered material remain unknown.

• Trip / restart clearing - PARTIALLY RESOLVED. Chute blockages and mechanical/electrical trips are confirmed initiating events, and material may be manually removed and deposited beside the conveyor or at the nearest safe location. The official written procedure and formal reintroduction method remain unknown.

• Chronic versus event-driven classification - RESOLVED AT WORKING LEVEL. Carryback, CV09 wet carryback, CV15 accumulation, sample-conveyor containment loss and screen leakage are treated as chronic/continuous working conditions. Trip/restart clearing is event-driven. Several alignment-related conditions remain of uncertain recurrence because live operating history was not fully observed.

# 17. Questionnaire Source Status

The contextual notes now incorporate two iterative source records: Questionnaire V1, which established the asset-by-asset spillage baseline, and Questionnaire V2, which interrogated mechanisms, contributing conditions, existing controls and working remedial concepts.

• Questionnaire V1 remains the primary source for asset-level spillage type, physical location, qualitative extent and the original photograph/video evidence index.

• Questionnaire V2 is the primary source for the developed carryback/cleaner mechanism, CV06 and CV09 cleaner findings, sample-conveyor loading/containment reasoning, belt-tracking follow-up, screening-water containment, structural-condition follow-up, housekeeping/recovery practices and trip/restart-generated spillage.

• Where V2 responses contain ambiguous checkbox semantics or internal inconsistencies, the written explanatory response and the earlier asset evidence are used to preserve the intended meaning without overstating certainty. The specific normalisation decisions are recorded in Section 23.

• The original completed questionnaires must be retained as source documents and evidence indexes. This running contextual document is the consolidated engineering narrative intended to become the primary drafting source for the final report.

18. Questionnaire V2 Findings - Mechanisms, Contributing Conditions and Existing Controls

Questionnaire V2 moves the contextual assessment from the V1 baseline of "what and where" into the mechanism-focused questions of "how, why, what currently controls it, and what should change". The findings below are consolidated from the completed V2 response. Confirmed observations, working interpretations and unresolved items are deliberately distinguished.

18.1 Product Life Cycle, Fines Generation and Carryback Development

• Fines are already present in the material after blasting and are further generated during primary crushing underground. The combined product then enters the conveyor ecosystem at the crusher/UC conveyor system and is transferred through successive conveyor sections until it reaches the surface screening plant.

• Carryback is confirmed as a recurring condition across multiple conveyor sections. In a typical conveyor section it becomes evident from the head/discharge side, where residual material passes onto the return belt after the primary product discharge.

• Residual fines progressively detach from the return belt and are additionally dislodged by return rollers/idlers, causing secondary deposits on conveyor frames, support structures and the surrounding environment.

• Wet fines were identified as more persistent carryback than dry material. Water can enter the product stream from several non-fixed operating sources, including dust-suppression sprays, general water-hose cleaning and, in some underground locations, high-pressure water used to assist the movement of blocked product in storage/silo areas. These examples describe observed or known practices; they are not asserted to occur continuously at every location.

18.2 System-Wide Belt-Cleaner / Scraper Deficiencies

• Observed or identified cleaner deficiencies include missing blades, excessive blade wear, incomplete blade coverage, inadequate belt contact pressure, reliance on manual adjustment, damaged/displaced support hardware, cleaners positioned outside the discharge containment envelope and removed material not being directed into the intended discharge path.

• The inspected conventional cleaner arrangements do not employ the proposed spring/self-tensioning mechanism. The absence of self-tensioning means blade wear can progressively reduce contact pressure and create a gap through which fines pass onto the return belt.

• Wet or sticky material can overwhelm the existing cleaner arrangement, and belt-surface condition may further reduce cleaning effectiveness.

• V2 supports the conclusion that both design/arrangement deficiencies and maintenance deficiencies contribute materially. The relative contribution varies by location.

• The mine operates on a reported four-days-operating / four-days-maintenance cycle. Maintenance responsibility is understood to rest with the designated section artisans and section supervisors. The formal belt-cleaner inspection standard, blade replacement criteria and maintenance records were not reviewed during the audit.

• The poor condition of some observed cleaners supports concern regarding inspection/replacement effectiveness, but it does not by itself establish the mine-wide formal maintenance frequency or procedure.

18.3 CV06 - Cleaner Maintenance / Support Failure

• CV06 was not fully functional at the time of inspection. CV06-REF-004 records the scraper support bush displaced from its housing and CV06-REF-005 records the head scraper blades missing.

• The displaced support bush would reduce or destabilise scraper contact with the belt, while the missing blades allow carryback to pass the head-cleaning point.

• The probable reason recorded for the displaced bush is poor housing design. The reason the blades were missing is unknown because no clear physical evidence of a blade-failure event was identified.

• Inadequate maintenance/replacement is treated as a probable contributing condition, but the exact failure history and previous maintenance history remain unknown.

• No separate CV06-specific redesign has been defined. The broader working concept is to prove an improved cleaner/support/tensioning arrangement at CV09 and, if successful, assess standardisation at other applicable head-discharge sections.

18.4 CV09 - Cleaner Location and Discharge-Chute Containment

• CV09 provides the strongest direct example of a cleaner arrangement that creates spillage even when material is successfully removed from the belt. The cleaner is outside the effective discharge-chute containment envelope, so removed wet carryback can fall directly into the surrounding environment.

• Residual carryback remains on the belt downstream of the cleaner and is further removed by return rollers, redistributing wet fines beyond the immediate discharge point.

• Questionnaire V2 records that physical space appears available to extend/revise the chute so that the cleaner can be enclosed while retaining maintenance access.

• The preferred working concept is a spring-loaded/self-tensioning cleaner that maintains positive belt contact as the wear element reduces in thickness, together with a mechanical travel/wear limiter that prevents over-travel toward the belt once the wear limit is reached.

• A primary-plus-secondary cleaner arrangement may be required and should be evaluated during detailed design/testing.

• For CV09, material removed from the belt should be directed into the CV09 discharge chute so that it joins the fines stream being conveyed/collected for the fines stockpile, rather than being released into the surrounding plant area.

• The need for additional drainage within the revised enclosure was not identified by the inspection response; the detailed chute design should nevertheless account for the actual wet-material behaviour observed during the pilot assessment.

18.5 Applicability of a Standard Cleaner Concept Across the Conveyor System

• Direct visual verification of cleaner presence/configuration was not possible at every conveyor. W10, CV01, CV02, CV04, CV14, L1, L3, Sample Conveyors 1 and 2, the UC05-UC01 system, U3, U5A and U5B were therefore marked Unknown for cleaner presence in the V2 matrix.

• Because these conveyors exhibit similar carryback behaviour and the discharge chutes are understood to use a broadly common manufacturing/layout approach, V2 records an engineering inference that similar cleaner-design limitations may exist at multiple locations. This inference is plausible but must not be presented as a directly observed defect at each unverified conveyor.

• CV09 and CV15 were identified as locations where a cleaner was observed and judged ineffective. CV06 had no functional cleaner at the time of inspection because the blades were missing.

• U4 is an explicit exception to the proposed common cleaner redesign because it uses a different/custom cleaner arrangement associated with its silo discharge configuration. UGU4-REF-011 and UGU4-REF-012 are identified in V2 as additional references for that different arrangement.

• A successful CV09 pilot would therefore justify a structured applicability review at other head-discharge points rather than automatic one-for-one installation without verification.

18.6 Sample Conveyors 2 and 3 - Loading Capacity and Side Containment

• The sample conveyors are continuously exposed to a combination of relatively large product lumps and product volumes that are high relative to the narrow usable belt width. The existing fixed chute geometry and upstream throughput influence the feed, but the exact mechanism available for controlling sample-product throughput remains unclear.

• No adjustable gate/slide-gate control was identified. V2 indicates that a mechanical redesign of the feed-control arrangement is likely to be required, but the specific throttling/metering solution remains open pending inspection of the existing feed arrangement.

• A working engineering hypothesis is that product which falls from the narrow belt can accumulate beneath the conveyor covers/structure and may then push or constrain the belt laterally, aggravating mistracking and further reducing usable carrying width. This mechanism has not been proven and requires inspection while the conveyors are in representative operating condition.

• The proposed secondary containment concept is a conveyor side skirt / side-containment skirting system using a fixed side-support structure with flexible rubber skirting. Its purpose is to retain large lumps near the belt edge and help keep the product stream within the usable carrying width.

• V2 proposes that the containment extend for a substantial length, potentially the full conveyor length, rather than only the loading zone. Approximate values of 250 mm height and 5-10 mm belt/structure clearance were recorded as preliminary concepts only and must not be treated as final design dimensions.

• The design should remain maintainable/removable, should not normally contact the belt, must allow normal tracking clearance and must avoid trapping product between the belt and the side containment.

• Side containment is a secondary control. Feed-rate/product-loading correction and belt-tracking correction remain the primary controls where those conditions are present.

18.6.1 Sample Conveyor 2

• Excessive product quantity and oversize lump dimensions are confirmed contributors to spillage. The V2 responses regarding off-centre loading/mistracking are internally inconsistent; the written follow-up states that head/tail pulley alignment and tensioning have not been verified.

• Accordingly, belt mistracking or off-centre loading at Sample Conveyor 2 must be treated as unconfirmed until the head and tail pulley alignment, belt tension and live tracking behaviour are checked.

• Feed throttling/metering should be the first control investigated, with side skirting considered as an additional retention measure.

18.6.2 Sample Conveyor 3

• Sample Conveyor 3 has the same product-size / belt-capacity concern as Sample Conveyor 2 and additionally shows significant belt mistracking.

• The available photographic evidence was interpreted as showing a lateral offset approaching approximately half of the usable belt carrying width. This is a qualitative visual estimate, not a measured tracking deviation.

• The mistracking was not observed live over sufficient time to determine whether it is constant, intermittent or load-dependent.

• Possible mechanical contributors include head/tail pulley alignment and belt tensioning. A further working hypothesis is that trapped spilled product within the enclosed conveyor may push the belt laterally. Neither mechanism has yet been quantified or proven.

• The combined working approach is to clear accumulated product, inspect and correct alignment/tensioning, observe live tracking under representative load, evaluate feed throttling/metering, and only then finalise the side-containment geometry.

18.7 L3 Conveyor - Tail-End Offset / Mistracking

• V2 records active lateral belt offset at L3 and describes the offset as greater than approximately 50% of the belt width. This is a qualitative estimate rather than a surveyed measurement.

• The condition was reported as continuously present during operation, but the root mechanical cause remains unknown. Tail-end adjustment bolts were suspected but not verified.

• V2 marked the offset as contributing directly to spillage; however, V1 recorded dry fines/carryback at L3 and did not record solid-product or mixed-product spillage. In addition, the V2 evidence field mistakenly referenced the Sample Conveyor 3 photograph series.

• For report purposes the reliable evidence set remains LoadoutStationL3-REF-001 through LoadoutStationL3-REF-008, with LoadoutStationL3-REF-001 documenting the tail-end offset. The relationship between the L3 offset and current solid-product loss should remain qualified until verified.

18.8 U4 Conveyor - Tripper Car Alignment

• V2 indicates that the tail-end pulley misalignment is fixed relative to the tripper-car structure and does not change as the car travels.

• Product loss from one side of the belt is repeatable when the tripper car reaches certain positions, but the position at which the condition is worst was not determined.

• The underlying source of pulley misalignment remains unknown. Track/rail alignment, bearing/mounting position, structural geometry, take-up behaviour and idler/training effects were not sufficiently verified to assign causation.

• The response indicates that the pulley can be mechanically realigned using existing adjustment provisions, although the detailed adjustment arrangement was not documented.

• After correction, tracking should be checked with the belt operating, under representative load where practical, and with the tripper car moved through its full travel range. The tail pulley, carrying path and return path should be included in the verification. The section supervisor was identified as the responsible verification role in the questionnaire.

18.9 Screening Plant - Water Escape and Structural Consequences

• Water loss of containment during normal screen/washing operation is now directly verified by Batch C videos, particularly Screen-REF-016 and Screen-REF-017, which show active spray/runoff escaping around the operating screen/chute structure. The earlier directional description of the upper-left side remains tied to the inspector's vantage point rather than being re-derived from the videos.

• The inspection could not safely approach the operating screen closely enough to determine whether the equipment-level cause is chute geometry, seals, enclosure height, spray direction, water flow, vibration, product build-up, corrosion/wear or a combination of these factors.

• Wet fines/slurry accumulate on floors and structural members around the escape side. Batch C photographs directly verify a severe wet/corrosive environment, including extensive corrosion and deterioration at screen/deck-support interfaces. Repeated wetting is therefore visually supported, although the media does not quantify the proportion of deterioration attributable to any single water source.

• No specific drainage defect was identified or communicated during the inspection. The primary unresolved issue is therefore the loss of containment itself rather than a confirmed drainage failure.

18.10 Damaged / Cut Screen Supports

• The contextual inspection identifies the affected screen-support members as load-bearing. Batch C photographs directly verify multiple physical cut-outs/notches, local openings and severe corrosion/deterioration in the affected members. Structural load path and remaining capacity cannot be established from photographs alone.

• The reason for the cuts is not confirmed. The previous suggestion that the members may have been cut to assist screen installation/replacement is the inspector's own working hypothesis based on past experience; it was not reported by mine personnel and is not supported by documentary evidence.

• No completed formal structural assessment was confirmed, no repair/replacement work was known to have started, and no temporary access/operational restriction was identified during V2.

• Whether replacement support geometry needs to change for installation clearance remains unknown and must be quantified if that option is pursued.

• Any change to the position or geometry of a load-bearing support requires structural calculation, verification and approval by an appropriately qualified structural authority.

18.11 L1 Conveyor / Tunnel - Structural Damage and Corrosion

• Batch C visual review strongly supports previous/intermittent belt-to-structure contact. LoadoutStationL1-REF-007 shows distinct smooth wear slots/notches in angular supports immediately adjacent to the running belt path. At the instant photographed, the belt is running clear of the supports rather than visibly rubbing them.

• Belt mistracking remains the probable cause of the earlier structural contact based on the geometry and inspection record. The still photograph does not prove the full dynamic root cause. Because the captured belt is clear of the supports, continuous current contact should not be claimed.

• Corrosion is present on the same general structures but is treated as a separate mechanism from mechanical belt-contact damage. LoadoutStationL1-REF-003 through -006 show severe concrete deterioration/spalling and exposed reinforcement, while LoadoutStationL1-REF-008 and -009 show severe steel section loss/perforation with corrosion. Fines accumulation can retain moisture around supports and water/wet-product exposure contributes to the corrosion environment, but the photographs do not quantify the relative contribution of each moisture source.

• The affected support structure requires repair/assessment, but V2 did not confirm that a formal structural assessment had already been completed or that repair work had been scheduled.

18.12 Housekeeping, Recovery and Rehandling

• Spilled material is removed using different methods depending on location. Confirmed examples include manual shovelling, wheelbarrow/hand-trolley handling and water wash-down. Vacuum/suction and dedicated recovery conveyors/chutes were not identified as normal methods.

• Clearing appears to be largely reactive/as-required when accumulation becomes excessive. Typical shift/daily/weekly frequencies were not established.

• Housekeeping removes the accumulation but recurring source mechanisms cause the same spillage to return. Current housekeeping resources were judged insufficient relative to the recurring spillage load, supporting the need to prioritise source control.

• Confined-space requirements materially complicate cleaning in some underground areas. The exact access constraint varies by location.

• Recovered fines/product are believed to be reintroduced onto the nearest practical belt section, but this requires confirmation. The official mine standard/procedure for housekeeping and product recovery was not established during the audit.

18.13 Conveyor Trips / Restart Preparation

• Trip/restart clearing is an episodic/event-driven spillage mechanism and is less frequent than chronic carryback.

• Chute blockages and mechanical/electrical conveyor trips were identified as typical initiating events. Overloading was not identified as a common initiating event in V2.

• Where a loaded belt cannot be restarted effectively, product may be manually removed with shovels and placed alongside the belt or at the nearest safe location to reduce restart load.

• The decision to remove product is understood to be controlled/authorised by a defined operational role, but the written work instruction was not reviewed.

• Removed product is later expected to be manually reintroduced into the process, but the official method and exact frequency were not confirmed. No designated temporary containment/laydown area was identified.

19. Consolidated Cross-System Cause Assessment After Questionnaire V2

The evidence now supports separation of the audit findings into confirmed mechanisms, probable/working causes and unresolved causes. This distinction should be retained when the final report is drafted.

19.1 Confirmed / Directly Supported Mechanisms

• Fines carryback occurs across multiple underground and surface conveyor sections and is progressively redistributed from the return belt onto conveyor structures and surrounding areas.

• CV06 had a non-functional cleaner condition, including a displaced support bush and missing scraper blades.

• CV09 has a cleaner outside the effective discharge containment, allowing removed wet carryback to become spillage; residual carryback then continues downstream.

• Sample Conveyors 2 and 3 have a product-size / product-volume relationship that exceeds the practical containment capability of the narrow belt arrangement. Sample Conveyor 3 additionally exhibits substantial mistracking.

• U4 belt mistracking and associated one-side lateral product loss are visually confirmed; the recorded tail-pulley misalignment remains the working upstream mechanical cause pending quantitative alignment verification.

• Screen process water escapes during normal operation and contributes to wet-fines accumulation and a corrosive structural environment.

• Load-bearing screen support members are cut/damaged.

• L1 shows physical evidence of belt-to-structure contact and structural corrosion.

• Housekeeping is repeatedly removing the consequence of recurring spillage while source mechanisms remain active.

• Trip/restart clearing is a distinct event-driven mechanism requiring separate operational handling from chronic carryback.

19.2 Probable / Working Causes Requiring Verification

• A common cleaner/chute design limitation may exist at multiple conveyors that were not visually accessible. This is inferred from common carryback behaviour and broadly similar discharge geometry, not directly verified at every location.

• The absence of self-tensioning in the conventional cleaner arrangement is a probable contributor to loss of contact pressure as blades wear.

• CV06 support-bush displacement is probably influenced by the housing design; the exact failure mechanism has not been independently tested.

• Sample-conveyor feed control likely requires redesign, but the actual control point and metering mechanism remain unknown.

• At Sample Conveyor 3, head/tail alignment, belt tensioning and/or trapped spillage beneath the enclosed belt may contribute to mistracking. These remain hypotheses pending live inspection.

• L3 shows significant offset, but the root cause and causal link to solid-product spillage remain unresolved.

• The equipment-level cause of screen-water escape remains unresolved because inspection access was limited during operation.

• The reason the screen supports were cut remains unresolved and must not be represented as known.

19.3 Systemic Versus Local Causes

• Systemic working causes: ineffective belt-cleaner/scraper design, variable cleaner maintenance condition, incomplete containment at some discharge points, and recurring fines carryback throughout the conveyor ecosystem.

• Local/asset-specific causes include the CV09 cleaner location outside containment; the CV06 missing blades/displaced support bush; U4 recorded tail-pulley misalignment together with visually verified mistracking; Sample Conveyor 3 severe mistracking combined with limited usable belt width; screen-water loss of containment; and L1 intermittent belt-to-structure contact.

• Feed-rate/loading control at the sample conveyors is a confirmed area of concern, but the exact control mechanism is not yet known well enough to assign a final root cause.

• Structural deterioration functions both as a consequence of long-term spillage/moisture exposure and as an independent maintenance/safety concern requiring dedicated assessment.

20. Updated Working Remedial Concepts After Questionnaire V2

The following are still working engineering concepts rather than issued-for-construction designs or final report recommendations. They provide the basis for the conceptual illustrations and later recommendation wording.

20.1 CV09 Pilot Belt-Cleaner / Chute Modification

• Use CV09 as the preferred pilot location for an improved cleaner arrangement because the current failure mechanism is directly observed and the location appears to have space for chute modification.

• Extend/revise the discharge-chute enclosure so that the cleaner operates inside the effective containment envelope and removed wet fines are directed into the CV09 fines-discharge stream.

• Use a spring-loaded/self-tensioning cleaner arrangement to maintain positive belt contact as the blade/wear element reduces in thickness.

• Provide positive wear/travel limiters so the cleaner cannot continue moving toward the belt after the wear element reaches its allowable limit.

• Evaluate whether primary and secondary cleaner stages are required for the wet/sticky service condition.

• Retain safe maintenance access for inspection, adjustment and replacement.

• Test effectiveness before considering standardisation elsewhere.

20.2 Wider Cleaner Standardisation

• If the CV09 pilot proves effective, survey the cleaner/chute arrangement at each applicable head-discharge point before rollout.

• Confirm cleaner presence, physical space, chute geometry, material condition and maintenance access at each location.

• Do not automatically apply the common design to U4 because its discharge and cleaner arrangement is materially different.

• Develop a common inspection/replacement standard covering blade condition, contact pressure, support hardware and wear limits if mine maintenance requirements are found to be undefined or inconsistent.

20.3 Sample Conveyors 2 and 3

• Inspect and quantify the actual feed-control arrangement and determine how the incoming sample-product volume can be throttled/metered.

• Measure representative product size relative to usable belt width/capacity and confirm whether the existing belt arrangement is suitable for the duty.

• Inspect head/tail pulley alignment, belt tensioning and live tracking, particularly on Sample Conveyor 3 and as a verification item on Sample Conveyor 2.

• Remove accumulated trapped product before alignment/tracking assessment so the belt can be evaluated in an unconstrained condition.

• Develop a side-support / rubber-skirt containment system as a secondary control. The current 250 mm height and 5-10 mm clearance values are preliminary only and require design verification.

• Finalise the side-containment geometry only after feed and tracking issues have been evaluated/corrected.

20.4 Belt Tracking / Alignment

• L3: verify the magnitude and source of the tail-end offset using the correct L3 evidence set and a physical alignment check; do not assume the current offset is the source of solid-product loss until confirmed.

• U4: mechanically realign the tail pulley using the available adjustment provisions, then verify belt centring through the full tripper-car travel range under representative operating conditions.

• Use fixed/repeatable reference points where practical so post-adjustment tracking can be trended rather than judged only visually.

20.5 Screen Water Containment and Structural Supports

• Perform a close inspection during a suitable safe shutdown to identify the exact water escape path and determine whether the primary cause is sealing, chute geometry, spray arrangement, flow, wear, movement or another condition.

• Modify the screen/chute containment only after the actual leak mechanism is confirmed.

• Obtain a formal structural assessment of cut/damaged load-bearing screen supports and define repair/replacement requirements.

• If support geometry is changed to improve future screen installation clearance, quantify the required clearance and obtain structural calculation/approval before implementation.

20.6 L1 Structural Condition

• Obtain structural assessment/repair definition for the damaged support members and corrosion.

• Retain belt-tracking monitoring because the observed structural slots are consistent with previous/intermittent belt contact even though the belt was centred during the latest inspection.

• Address fines/moisture accumulation around support interfaces as part of the corrosion-control approach.

20.7 Housekeeping and Trip/Restart Handling

• Prioritise source-control improvements so housekeeping is not the principal control for recurring carryback.

• Confirm the mine's actual housekeeping and recovered-product reintroduction procedure, including responsibility and normal clearing frequency.

• Confirm the written trip/restart belt-unloading procedure and assess whether defined temporary containment/laydown locations would reduce secondary handling and uncontrolled deposits.

21. Evidence Status, Engineering Limitations and Qualification of Findings

• The audit did not include direct access to every belt cleaner/discharge chute. System-wide cleaner-design conclusions outside the verified examples must therefore be presented as an engineering inference until local configuration is checked.

• Formal maintenance schedules, belt-cleaner inspection criteria, maintenance records and OEM cleaner standards were not reviewed. The observed condition can support maintenance concerns but cannot establish the full maintenance-management history.

• The sample conveyors were not observed live for sufficient time to distinguish constant, intermittent and load-dependent mistracking. Proposed causal mechanisms involving trapped spillage and belt displacement remain working hypotheses.

• Approximate tracking descriptions such as "about 50% of belt width" are visual estimates and not surveyed measurements.

• The preliminary sample-conveyor side-skirt dimensions (approximately 250 mm height and 5-10 mm clearance) are conceptual values and require measurement/design verification before use in a final recommendation or fabrication drawing.

• The screen-water escape was observed while equipment was operating, but safe access did not allow close identification of the equipment-level leakage mechanism. The report should state the observed loss of containment and recommend targeted investigation rather than assigning an unsupported specific defect.

• The explanation for why the screen supports were cut is not confirmed and originates from the inspector's own hypothesis. It must not be presented as a reported mine practice or established cause.

• L3 V2 evidence text referenced Sample Conveyor 3 photographs in error. The L3 evidence index remains LoadoutStationL3-REF-001 through LoadoutStationL3-REF-008.

• The audit philosophy remains qualitative/mechanism-focused. No attempt has been made to calculate total spillage mass/volume for each section.

• Repository evidence availability has been reconciled against the contextual reference ranges. Independent visual review is now complete for all six planned batches, covering all 23 evidence groups / 197 media files in the repository. Photograph/video-specific classifications are recorded in Sections 27 through 37 and in the corresponding Report-Working/evidence_visual_review_batch_A.md through _F.md files. Remaining uncertainty now relates to engineering verification, measurements, procedures and design development rather than unreviewed evidence media.

22. Remaining Targeted Follow-Up Items

Questionnaire V2 is sufficiently complete for integration into the report-development context. A broad Questionnaire V3 is not currently required. Remaining work should be targeted to the genuine technical gaps below.

• Develop a contextual/conceptual drawing of the proposed CV09 cleaner/chute pilot, including self-tensioning contact, wear/travel limits, maintenance access and the possible primary/secondary cleaning arrangement.

• Develop a contextual/conceptual drawing of the Sample Conveyor side-support / rubber-skirt containment concept.

• Measure/verify CV09 chute space and cleaner geometry before any pilot design is treated as final.

• Verify cleaner presence/configuration at conveyors where V2 recorded Unknown before a system-wide standard is formally recommended.

• Inspect the Sample Conveyor feed arrangement, measure product size versus usable belt width/capacity, and verify head/tail pulley alignment, belt tension and live tracking.

• Verify L3 alignment using the correct L3 evidence set and determine whether the offset is currently causing solid-product loss.

• Identify the mechanical source/adjustment details for U4 tail-pulley misalignment and complete post-correction full-travel tracking verification.

• Inspect the screen/chute closely during a safe shutdown to identify the actual water escape mechanism.

• Obtain/confirm structural assessment and repair planning for the cut screen supports and L1 structural damage/corrosion.

• If the final report is to make procedural recommendations, confirm the mine's housekeeping/recovery and trip/restart unloading work instructions rather than relying on inferred practice.

23. Questionnaire V2 Integration and Normalisation Record

The following interpretation rules were applied while consolidating the completed V2 response so that checkbox wording does not create conclusions contrary to the accompanying written explanation or V1 evidence.

• Spring/tensioning mechanism: V2 checkbox semantics were contradictory, but the written note explicitly states that the current conventional designs do not employ spring mechanisms. The contextual notes therefore record self-tensioning as absent from the inspected conventional arrangement and as part of the proposed redesign.

• Sample Conveyor 2: V2 contains conflicting answers regarding off-centre loading/mistracking. The written follow-up states that pulley alignment/tensioning were not verified. Mistracking at Sample Conveyor 2 is therefore retained as unconfirmed.

• L3: V2 marked the offset as contributing to spillage but referenced Sample Conveyor 3 photos in the evidence field, while V1 did not record solid-product spillage at L3. The contextual notes retain active offset/mistracking as observed but keep its contribution to solid-product loss qualified pending verification.

• Tracking verification: despite an Unknown checkbox for carrying-idler verification, the written V2 method explicitly states that carrying-idler alignment should be verified. This is retained as a proposed verification step, not as a completed check.

• Sample feed control: product quantity/size relative to belt containment capacity is retained as a confirmed concern. The exact feed-control mechanism and the most suitable throttling/metering method remain unknown.

• Screen supports: the possibility that supports were cut to assist installation is retained only as the inspector's hypothesis, not as reported or confirmed fact.

• U4 evidence index: UGU4-REF-011 and UGU4-REF-012 are added as V2-described references showing the different/custom U4 cleaner arrangement.

Context readiness: Questionnaire V1 and Questionnaire V2 have now been integrated into the running contextual narrative. The document contains a sufficiently developed evidence/cause framework to begin drafting report sections, while the remaining items in Section 22 can be resolved through targeted verification and conceptual engineering illustrations.

24. Repository Evidence Inventory Reconciliation

Following consolidation of Questionnaire V1 and Questionnaire V2 into Contextual Notes V3, the evidence repository was inventoried and cross-checked against the evidence references used throughout this document. The purpose of this reconciliation is to confirm that the referenced source files are present and traceable before the detailed visual-review and report-population stages begin.

24.1 Repository Inventory Status

• Repository: Speedie007/Spillage-Report, branch main.

• Primary evidence root: Evidence/.

• Evidence groups / inspected asset groups inventoried: 23.

• Total media files inventoried: 197, comprising 189 photographs and 8 videos.

• All evidence ranges and specific reference IDs currently cited in Contextual Notes V3 are represented in the repository inventory. No contextual evidence set is currently recorded as missing.

• This reconciliation confirms file presence and reference traceability only. It does not by itself verify what each image/video visually shows.

24.2 Reconciliation by Plant Area

• Surface Screening Section: W10 (12 files), CV01 (4), CV02 (4), CV03 (3), CV04 (5), CV06 (5), CV09 (8), CV14 (2), CV15 (8), and Screens / Screen Chutes (17) are represented.

• Surface Loading Station: L1 (18 files), L2 (7), L3 (8), L4 (8), Sample Conveyor 1 (11), Sample Conveyor 2 (8), and Sample Conveyor 3 (13) are represented.

• Underground Central Block: Crusher (5 files) and the combined UC05 / UC04 / UC03 / UC02 / UC01 conveyor-system evidence set (16) are represented.

• Underground Secondary Plant: U3 (8 files) and U4 Conveyor - Tripper Car (12) are represented.

• Underground Silos / Feeders: U5A (9 files) and U5B (6) are represented.

24.3 Filename and Reference-Control Items

• U4 video reference UGU4-REF-010 has been normalised in the repository. The earlier filename UGU4-REF-0010.MP4 was renamed to UGU4-REF-010.MP4 so that the repository filename now matches the canonical contextual reference.

• CV09 video files retain mixed-case repository filenames (Cv09-Ref-006.mp4, Cv09-Ref-007.mp4 and Cv09-Ref-008.mp4). The canonical report/evidence references remain CV09-REF-006, CV09-REF-007 and CV09-REF-008.

• L2 contains both LoadoutStationL2-REF-005.jpg and LoadoutStationL2-REF-005.mp4. The shared numeric reference is therefore ambiguous unless the media type or exact repository path is stated. No renumbering has yet been imposed.

• The additional U4 cleaner/scraper photographs UGU4-REF-011 and UGU4-REF-012 are present in the repository and remain mapped to the different/custom U4 cleaner arrangement identified in Questionnaire V2.

24.4 Evidence Interpretation Status After Reconciliation

• At the repository-reconciliation stage, evidence references were initially classified as Context-mapped / Visual verification pending. All six planned visual-review batches are now complete. Important evidence is classified as Direct, Supporting, Context only, or source-recorded/mechanism information according to what the media independently demonstrates.

• As each visual-review batch is completed, important items are classified as Direct, Supporting, or Context only according to what the image/video independently demonstrates; source-recorded inspection observations remain separately identified where the media does not prove the complete mechanism.

• Report-Working/evidence_index.md has now been final-consolidated following completion of Batch F. It records completed review status, the applicable batch-review file, first-draft figure selections and key evidence qualifications for all 23 asset/evidence groups.

• Final report figure selection should favour the smallest set of photographs/videos that clearly demonstrate each finding while retaining the complete original evidence set in the repository.

25. Planned Visual Evidence Review Sequence

The visual-review stage should be completed in focused evidence batches. Priority is given first to the locations that support the strongest causal, design, structural or remedial conclusions, followed by the broader system-condition evidence.

25.1 Priority Review Batches

• Batch A - Cleaner / alignment mechanisms: CV06, CV09 and U4 Conveyor - Tripper Car. COMPLETED. The visual review confirms the key cleaner-maintenance and cleaner/chute examples and visually verifies U4 belt mistracking / one-side spillage, while retaining tail-pulley misalignment as a source-recorded observation requiring quantitative verification.

• Batch B - Sample-conveyor loading / containment / tracking: Sample Conveyor 2, Sample Conveyor 3 and L3. COMPLETED. The review retained Sample Conveyor 2 mistracking as unconfirmed, visually supported Sample Conveyor 3 lateral offset / mistracking without assigning a measured magnitude, and retained the L3 solid-product-spillage causal link as unconfirmed.

• Batch C - Structural / water-loss evidence: Screens / Screen Chutes and L1. COMPLETED. The review directly verified screen-water loss of containment, severe screen-area corrosion / cut support members, and L1 structural deterioration / previous belt-contact evidence.

25.2 Secondary Review Batches

• Batch D - Remaining Surface Screening conveyors: W10, CV01, CV02, CV03, CV04, CV14 and CV15. COMPLETED.

• Batch E - Remaining Loading Station evidence: L2, L4 and Sample Conveyor 1. COMPLETED.

• Batch F - Remaining Underground evidence: Crusher, UC05-UC01 continuous system, U3, U5A and U5B. COMPLETED.

26. Context Readiness After Repository Reconciliation

Questionnaire V1 and Questionnaire V2 are integrated, the evidence repository inventory is reconciled against the contextual reference structure, no referenced evidence group is currently missing, all six visual-review batches are complete, and Report-Working/evidence_index.md has been final-consolidated. The contextual evidence base is therefore ready to transition from evidence-review mode into report-template adaptation and first-draft report population. Findings must continue to preserve the distinction between direct observation, source-derived description, engineering interpretation and unverified hypothesis.

27. Visual Evidence Review - Batch A

The first independent visual-evidence review was completed using the uploaded CV06, CV09 and U4 Conveyor - Tripper Car photographs/videos. The review compared what the media itself demonstrates against the existing Questionnaire V1/V2 and Contextual Notes V3.1 narrative. Evidence was classified as Direct, Supporting or Context only, with source-recorded inspection observations retained separately where the media does not independently prove the full mechanism.

27.1 CV06 - Visual Verification

• CV06-REF-004 directly shows the scraper-support bush/insert displaced or protruding outboard relative to the orange support housing. This visually verifies the recorded support-bush displacement.

• CV06-REF-005 directly shows the cleaner support beam/cross-shaft beneath the belt with scraper blade elements absent from the working belt width. This visually supports the recorded condition that the head scraper blades were missing and the cleaner was not functional as intended.

• CV06-REF-001 to -003 are principally context/orientation evidence and are not required to prove the cleaner defect.

• The visual record does not establish why the support bush displaced or why the blades were absent. Poor housing design therefore remains a working engineering assessment and the blade-loss/removal history remains unknown.

• Preferred first-draft report photographs: CV06-REF-004 and CV06-REF-005.

27.2 CV09 - Visual Verification

• CV09-REF-002 directly shows wet-fines/slurry accumulation and standing wet material on the floor below the conveyor/transfer region.

• CV09-REF-003 directly shows the cleaner/scraper assembly immediately below the belt, heavily coated with wet fines/carryback. In the supplied view, the cleaner region is visibly open to the surrounding structure rather than enclosed within a complete local containment envelope.

• CV09-REF-004 and CV09-REF-005 support the wider fallout/redistribution condition by showing fines/material deposits on structural members and the area below the conveyor.

• CV09-REF-006 video provides direct dynamic evidence of active droplets/wet material falling from the overhead conveyor/transfer region to the wet floor below. The video verifies that the wet-spillage condition was active rather than only a historical deposit.

• CV09-REF-007 provides supporting walk-through evidence of deposits and cleaner-area context. CV09-REF-008 provides close supporting/direct evidence of the cleaner region and substantial wet-fines build-up.

• The visual evidence materially strengthens the working conclusion that material removed at the cleaner must be captured and returned to the intended CV09 fines discharge path. However, final chute-extension geometry, physical dimensions, spring/self-tensioning design, travel limiter and primary/secondary cleaner arrangement remain engineering design concepts requiring site measurement and detailed design.

• Preferred first-draft evidence: CV09-REF-003, CV09-REF-002, video CV09-REF-006, with CV09-REF-008 as supplementary cleaner-area video evidence.

27.3 U4 Conveyor - Tripper Car Visual Verification

• UGU4-REF-005 and UGU4-REF-006 directly show very large accumulated fines/product deposits beside the tripper/conveyor structure, confirming the severity and persistence of local spillage.

• UGU4-REF-007 directly shows significant lump/solid product accumulated predominantly along one side of the conveyor/tripper area. The still photograph demonstrates the accumulated result of lateral product loss, rather than the instant at which material leaves the belt.

• UGU4-REF-009 directly shows the belt laterally displaced relative to the carrying-idler/support geometry. This visually verifies the mistracking condition. No measured percentage offset should be assigned from the photograph alone.

• UGU4-REF-004 supports the same mistracking condition from a wider longitudinal viewpoint.

• UGU4-REF-008 and UGU4-REF-010 video show the running tail-pulley/belt region and visually support the recorded pulley-alignment concern. However, the supplied visuals do not provide a fixed dimensional/survey reference sufficient to independently quantify the tail-pulley alignment relative to the tripper structure.

• Accordingly, belt mistracking and associated one-side spillage are visually verified. Tail-pulley misalignment remains a source-recorded direct inspection observation that is visually supported, but still requires quantitative alignment verification before being stated as a measured geometric condition.

• UGU4-REF-011 and UGU4-REF-012 directly confirm a distinct/custom transverse cleaner/scraper arrangement with substantial dedicated support structure. This visually supports treating U4 as an exception when considering wider standardisation of a conventional head-cleaner concept.

• Preferred first-draft evidence: UGU4-REF-009 for mistracking, UGU4-REF-007 for one-side solid-product spillage, optionally UGU4-REF-006 for fines-severity context, UGU4-REF-010 as supporting operating video, and one of UGU4-REF-011 / UGU4-REF-012 for the U4-specific cleaner arrangement.

27.4 Batch A Comparison Outcome

• No Batch A visual evidence materially contradicts the existing contextual narrative.

• CV06 and CV09 findings are strengthened by direct visual verification of the principal cleaner-maintenance/design and wet-spillage conditions.

• The principal refinement is evidentiary wording at U4: distinguish the visually verified mistracking/spillage from the inspector-recorded tail-pulley misalignment, which is visually supported but not independently quantified by the supplied media.

• The detailed item-by-item visual review and candidate captions are recorded in Report-Working/evidence_visual_review_batch_A.md.

28. Context Status After Batch A Visual Review

Repository inventory reconciliation and Batch A visual verification are complete. The current cause-and-effect framework remains valid, with the U4 evidence wording refined as described above. The next priority visual-review batch is Sample Conveyor 2, Sample Conveyor 3 and L3 Conveyor. This batch will test the loading-capacity, side-containment and mistracking interpretations, including the current distinction between confirmed Sample 3 mistracking, unconfirmed Sample 2 mistracking and the unresolved causal contribution of the L3 offset.

29. Visual Evidence Review - Batch B

The second independent visual-evidence review was completed using the uploaded Sample Conveyor 2, Sample Conveyor 3 and Loading Station L3 photographs. The review compared the media itself against the existing contextual narrative and retained the distinction between direct visual evidence, source-recorded inspection observations and engineering interpretation.

29.1 Sample Conveyor 2 - Visual Verification

• LoadoutStationSample2-REF-001 through -003 directly show extensive lump-product accumulation around the sample-conveyor installation and adjacent floor/access areas.

• LoadoutStationSample2-REF-004 directly shows coarse/lump product occupying a substantial proportion of the available narrow belt carrying width.

• LoadoutStationSample2-REF-005 through -007 show the loaded narrow belt beneath the local enclosure/cover arrangement, with coarse product concentrated within the limited carrying width and, in places, material accumulated outside the intended belt path.

• LoadoutStationSample2-REF-008 shows additional mixed-size material/fines accumulated on lower structure beneath the conveyor.

• The photographs strengthen the working conclusion that product size and local loading relative to the narrow belt are important containment concerns. They do not establish the live feed rate, the design-capacity loading limit, or the final upstream feed-control method.

• No photograph in this batch provides clear independent evidence of significant Sample Conveyor 2 mistracking. The current status therefore remains: mistracking unconfirmed and requiring verification.

• Preferred first-draft photographs: LoadoutStationSample2-REF-004 for product size versus belt width; REF-006 or -007 for containment loss; and REF-001 or -003 for overall spillage severity.

29.2 Sample Conveyor 3 - Visual Verification

• LoadoutStationSample3-REF-002, -008, -011 and -012 directly show substantial solid/lump-product spillage and accumulation outside the intended belt path and on surrounding structure/grating.

• LoadoutStationSample3-REF-005 and -006 directly show coarse/lump product occupying a substantial proportion of the narrow belt width.

• LoadoutStationSample3-REF-003 through -007 provide close-range supporting evidence of an asymmetric belt relationship to adjacent rollers/enclosure while also showing material outside the intended carrying path.

• LoadoutStationSample3-REF-013 provides the strongest tracking evidence. The belt is visibly asymmetric relative to the side/training-roller arrangement, with one belt edge operating close to the lower training/guide hardware and noticeably greater clearance on the opposite side.

• Accordingly, Sample Conveyor 3 lateral offset/mistracking is visually supported. The earlier approximate statement that about half of the usable carrying width may be lost remains a qualitative inspector estimate and must not be treated as a measured percentage or dimension.

• The photographs do not establish whether the mistracking is constant, intermittent or load-dependent, and they do not prove its mechanical root cause. Head/tail alignment, belt tension and trapped spilled product remain inspection hypotheses rather than established causes.

• Preferred first-draft photographs: LoadoutStationSample3-REF-013 for tracking; REF-005 or -006 for product size/loading; REF-002 for local containment loss; and REF-012 for the wider spillage/housekeeping consequence.

29.3 Loading Station L3 - Visual Verification

• LoadoutStationL3-REF-001 visually supports the inspector-recorded tail-end belt/pulley offset by showing an asymmetric relationship in the photographed tail-end area.

• Perspective and the absence of a fixed dimensional reference prevent a reliable percentage or millimetre offset from being derived from LoadoutStationL3-REF-001 alone.

• LoadoutStationL3-REF-007 directly shows fine-material coating/accumulation on the side guarding and adjacent structure.

• LoadoutStationL3-REF-008 directly shows fines and small product fragments accumulated on the lower corrugated surface/deck beneath the conveyor area.

• The remaining L3 photographs principally provide equipment, guarding and structural context.

• No Batch B photograph establishes a direct causal chain from the L3 tail-end offset to significant solid/lump-product spillage. This remains consistent with Questionnaire V1, which recorded dry fines/carryback at L3 but did not record solid/mixed-product spillage.

• Preferred first-draft photographs: LoadoutStationL3-REF-001 for the tail-end offset concern; REF-007 for fines/carryback contamination; and REF-008 as optional lower-area accumulation evidence.

29.4 Batch B Comparison Outcome

• No Batch B photograph materially contradicts the current contextual narrative.

• Sample Conveyor 2 product-size/loading-versus-belt-width and containment concerns are visually strengthened, while significant mistracking remains unconfirmed.

• Sample Conveyor 3 mistracking is visually supported, but any earlier percentage estimate of usable belt-width loss remains unmeasured and should be treated as qualitative only.

• L3 tail-end offset is visually supported; its direct causal contribution to solid-product spillage remains unconfirmed.

• Neither Sample Conveyor 2 nor Sample Conveyor 3 photographs establish design-capacity overload in a quantitative engineering sense. They demonstrate large/coarse product relative to the available narrow belt width and significant containment loss.

• The detailed item-by-item visual review and candidate captions are recorded in Report-Working/evidence_visual_review_batch_B.md.

30. Context Status After Batch B Visual Review

Repository inventory reconciliation and visual verification of Batches A and B are complete. The current cause-and-effect framework remains valid. The strongest evidence-based refinements are that Sample Conveyor 2 mistracking remains unconfirmed; Sample Conveyor 3 lateral offset is visually supported but unmeasured; and L3 remains an alignment/tracking corrective-action item without a currently demonstrated solid-product-spillage causal chain. The next priority visual-review batch is Screens / Screen Chutes and L1 Conveyor, where the principal objectives are to verify process-water escape, wet-fines exposure, structural corrosion, cut/damaged screen supports, L1 belt/structure contact evidence and related structural condition.

31. Visual Evidence Review - Batch C

The third independent visual-evidence review was completed using the Screens / Screen Chutes and Loading Station L1 photographs/videos. The review compared what the media independently demonstrates against the current contextual narrative and retained the distinction between direct visual evidence, source-recorded inspection identification and engineering interpretation.

31.1 Screens / Screen Chutes - Visual Verification

• Screen-REF-016 and Screen-REF-017 directly show active process-water spray/runoff escaping around the operating screen/chute structure and dispersing into the surrounding plant area. Screen-REF-015 provides additional active-wetting/runoff evidence.

• The videos directly verify loss of containment during operation, but do not resolve the internal equipment-level mechanism. Seal condition, chute geometry, spray direction, flow, vibration, product build-up and wear/corrosion therefore remain possible contributors requiring safe close inspection.

• Screen-REF-004, Screen-REF-008 and related photographs directly show severe corrosion and deterioration at screen/deck-support interfaces, strongly supporting repeated wetting exposure.

• Screen-REF-006 and Screen-REF-010 provide particularly strong direct evidence of substantial cut-outs/notches in support members. Similar local openings are also visible in Screen-REF-005 and Screen-REF-009.

• The inspection/contextual source identifies the affected members as load-bearing. The photographs verify the physical damage but cannot independently establish structural load path, remaining capacity or acceptability.

• No visual evidence establishes why the support members were cut. The earlier installation/replacement-clearance explanation remains an unverified inspector working hypothesis.

• Preferred first-draft evidence: Screen-REF-016 or -017 for active water escape; Screen-REF-004 or -008 for corrosion severity; Screen-REF-006 or -010 for cut/damaged support geometry; and Screen-REF-003 or -014 only if a wider orientation image is required.

31.2 Loading Station L1 - Visual Verification

• LoadoutStationL1-REF-001, -002, -010 through -015, and -017/-018 directly show extensive dry fines and lump/solid-product accumulation on floors, access areas and surrounding structure.

• LoadoutStationL1-REF-003 through -006 directly show severe tunnel/overhead concrete deterioration, including spalling, exposed reinforcement and corrosion staining.

• LoadoutStationL1-REF-007 provides the strongest belt-contact evidence. Smooth local wear slots/notches are visible in angular steel supports immediately adjacent to the running belt path. At the instant photographed, the belt is visibly clear of the supports.

• This directly strengthens the conclusion of previous/intermittent belt-to-structure contact while also supporting the qualification that continuous current contact should not be claimed.

• LoadoutStationL1-REF-008 and LoadoutStationL1-REF-009 directly show severe local steel section loss/perforation with corrosion. Remaining structural capacity cannot be determined visually.

• Belt mistracking remains the probable mechanism for earlier contact based on the geometry and inspection record, but the still photographs do not prove the full dynamic root cause.

• Preferred first-draft evidence: LoadoutStationL1-REF-007 for wear/contact history; REF-003 or -005 for concrete deterioration/exposed reinforcement; REF-008 for steel section loss; REF-013 for overall spillage severity; and optionally REF-017 for upper-structure material accumulation.

31.3 Batch C Comparison Outcome

• No Batch C evidence materially contradicts the current contextual narrative.

• The screen-water loss of containment is now independently visually verified by operational video rather than relying only on source-recorded observation.

• The severe screen-area corrosion and multiple physical cut-outs/notches in support members are directly visually verified.

• The L1 contact narrative is strengthened: wear slots are directly visible next to the belt path, while the operating belt is clear of the supports in the captured condition.

• The principal qualifications remain unchanged: exact screen leak mechanism, reason for screen-support cuts, structural remaining capacity and the full dynamic root cause of prior L1 contact all require further verification.

• Detailed item-by-item classifications and candidate captions are recorded in Report-Working/evidence_visual_review_batch_C.md.

32. Context Status After Batch C Visual Review

Repository reconciliation and visual verification of Batches A, B and C are complete. The evidence base now directly verifies the principal cleaner defects at CV06/CV09, U4 mistracking/one-side spillage, Sample Conveyor 3 mistracking, Sample Conveyor 2/3 loading-containment concerns, screen water loss of containment, screen structural deterioration/cut-outs, and L1 structural/contact conditions. Remaining Batches D-F primarily complete the broader system-condition record and provide representative evidence for the less complex conveyor locations. The next visual-review batch is W10, CV01, CV02, CV03, CV04, CV14 and CV15.

33. Visual Evidence Review - Batch D

The fourth independent visual-evidence review was completed using the W10, CV01, CV02, CV03, CV04, CV14 and CV15 photographs. This batch primarily completes the broader Surface Screening condition record and refines the distinction between directly visible physical conditions and source-recorded operating/mechanical observations.

33.1 W10 Conveyor - Primary Feed

• W10-REF-002, -003, -004, -005, -008 and -009 directly show substantial dry lump-product and mixed-size/fines accumulation within the enclosed tunnel.

• W10-REF-010 and -011 provide strong context for the narrow enclosed/restricted-access arrangement and show product accumulation along the side-access route.

• Dust/fines coating on the belt and surrounding structure supports the existing fines/carryback context, although not every deposit can be assigned specifically to carryback from still photographs.

• Individual images show locally severe piles, but the existing asset-level localised-to-moderate classification is retained rather than reclassified solely from close views.

• Preferred first-draft evidence: W10-REF-002 for spillage severity and W10-REF-010 or -011 for tunnel/restricted-access context.

33.2 CV01

• CV01-REF-003 and -004 directly show large wet/damp fines-slurry accumulations and standing water/slurry below the conveyor.

• CV01-REF-001 directly shows severe visible surface damage/abrasion on the identified return support roller.

• A still photograph cannot prove that the roller was mechanically seized at the time of inspection. The 'seized' condition therefore remains a source-recorded inspection observation unless separately verified dynamically/mechanically.

• Preferred first-draft evidence: CV01-REF-001 for roller damage and CV01-REF-003 or -004 for wet-fines/slurry accumulation.

33.3 CV02

• CV02-REF-001 directly shows substantial dry fines and mixed-size/lump-product accumulation below/adjacent to the transfer structure.

• CV02-REF-003 directly shows dry lump-product and fines around return-side/support rollers and lower structure.

• The existing localised-to-moderate asset-level classification remains consistent with the visual evidence.

• Preferred first-draft evidence: CV02-REF-001 and CV02-REF-003.

33.4 CV03

• CV03-REF-002 and -003 show a comparatively clean belt/transfer area with no significant visible spillage, supporting the Questionnaire V1 no-spillage baseline.

• This negative/baseline evidence is useful because it confirms that the inspection did not classify every conveyor as a spillage location.

• No CV03 photograph is required in the main report unless a clean comparator is useful.

33.5 CV04

• CV04-REF-002 directly shows saturated wet-fines/slurry accumulation with visible droplets/wet fallout in the photographed area.

• CV04-REF-004 provides a second strong close view of heavily saturated fines/slurry around conveyor equipment/support structure.

• CV04-REF-005 gives the clearest wider evidence of multiple accumulated fines piles beneath/around the conveyor and supports the extensive/widespread classification.

• Preferred first-draft evidence: CV04-REF-002 for wet-slurry condition and CV04-REF-005 for overall extent.

33.6 CV14

• CV14-REF-001 shows small dry-fines/product deposits along the access grating/walkway.

• CV14-REF-002 directly shows a localised fines/carryback mound beneath the return-side structure.

• The trace-to-localised classification remains appropriate; the photographs do not support escalation to a widespread condition.

• Preferred first-draft evidence: CV14-REF-002.

33.7 CV15

• CV15-REF-003 directly confirms the chute E-3181 identification used in the contextual notes.

• CV15-REF-004, -005, -006 and -007 directly show extensive fines/carryback deposits distributed across grating, cross-bracing and structural members around the conveyor support structure.

• CV15-REF-008 provides a strong close view of a large localised dry/dried-fines deposit on a structural/platform surface.

• The supplied still photographs primarily show dry or dried fines. The wet-fines component of the inspection classification is therefore retained as source-recorded information rather than being stated as visually obvious in this still set.

• The photographs show the discharge/cleaner region and contamination, but do not independently prove cleaner operational effectiveness. The existing assessment that the cleaner was observed and judged ineffective remains a source-recorded inspection conclusion.

• Preferred first-draft evidence: CV15-REF-003 for location traceability, CV15-REF-004 or -006 for widespread accumulation, and optionally CV15-REF-008 for a close deposit view.

33.8 Batch D Comparison Outcome

• No Batch D visual evidence materially contradicts the current contextual narrative.

• W10, CV02, CV04, CV14 and CV15 spillage/accumulation classifications are visually supported, while CV03 provides a useful clean baseline.

• The principal evidentiary refinements are that CV01 roller damage is visually verified but the seized state is not provable from a still image, and CV15 wet-fines/cleaner-effectiveness wording remains source-recorded rather than independently demonstrated by the still photographs.

• Detailed item-by-item classifications and candidate captions are recorded in Report-Working/evidence_visual_review_batch_D.md.

34. Context Status After Batch D Visual Review

Repository reconciliation and visual verification of Batches A through D are complete. The Surface Screening evidence record is now substantially visually verified, including W10 tunnel spillage, CV01 wet fines and roller damage, CV02 mixed dry spillage, CV03 clean baseline, CV04 wet-slurry accumulation, CV14 localised carryback and CV15 widespread fines accumulation around E-3181. The remaining visual-review work is Batch E (L2, L4 and Sample Conveyor 1) followed by Batch F (Crusher, UC05-UC01, U3, U5A and U5B).

35. Visual Evidence Review - Batch E

The fifth independent visual-evidence review was completed using the uploaded L2, L4 and Sample Conveyor 1 photographs, together with the L2 video. The review compared the media against the existing contextual narrative and retained the distinction between direct visual evidence, source-recorded inspection observations and mechanism interpretations.

35.1 L2 Conveyor - Visual Verification

• The L2 photographs support the Questionnaire V1 baseline that no significant spillage was observed at the inspected location. Access walkways, chute / transfer structures and surrounding equipment appear comparatively clean.

• The approximately 28-second LoadoutStationL2-REF-005.mp4 sequence is principally an equipment / guarding inspection and does not show significant active material loss or large existing spillage deposits during the captured period.

• This is a time-specific inspected-condition baseline and must not be interpreted as proving that L2 cannot spill under other operating states.

• LoadoutStationL2-REF-005.jpg and LoadoutStationL2-REF-005.mp4 remain separate evidence items sharing the same numeric reference; media type / exact path must be used to disambiguate.

• Normally no L2 figure is required in the main report unless a clean comparator is useful; LoadoutStationL2-REF-004 or LoadoutStationL2-REF-005.jpg would be suitable if used.

35.2 L4 Conveyor - Visual Verification

• LoadoutStationL4-REF-003 and LoadoutStationL4-REF-004 directly show localised dry fines and coarse / lump-product accumulation in lower channels / troughs and on structural members adjacent to the conveyor.

• The main access walkway is comparatively clean in LoadoutStationL4-REF-001 and LoadoutStationL4-REF-005, supporting retention of the trace-to-localised asset-level classification rather than escalation to widespread spillage.

• LoadoutStationL4-REF-007 and LoadoutStationL4-REF-008 show the operating loaded belt carrying coarse product generally within the belt trough, with residual material visible on adjacent support structure.

• No still image captures an active material-fall event; the photographs therefore verify the accumulated local spillage condition but do not independently establish the instantaneous loss mechanism.

• Preferred first-draft photographs: LoadoutStationL4-REF-003 or -004 for localised accumulation, with REF-007 or -008 as optional operating context.

35.3 Sample Conveyor 1 - Visual Verification

• The photographs strongly verify extensive / widespread accumulation distributed through the Sample Conveyor 1 installation rather than being confined to one floor-level location.

• LoadoutStationSample1-REF-001 and -002 directly show fines / mixed product around the conveyor base, stairs and access areas.

• LoadoutStationSample1-REF-004 and -007 show substantial deposits on multiple horizontal structural members at different elevations.

• LoadoutStationSample1-REF-005 and -006 show thick adhered / dried fines deposits with pronounced run / drip-like streaking. This supports a history of sticky / moisture-laden material deposition, but does not prove active wet material at the instant photographed or identify the moisture source.

• LoadoutStationSample1-REF-008 directly shows coarse / lump-product spillage distributed across the access grating beside the conveyor.

• LoadoutStationSample1-REF-010 directly shows a large mixed fines / lump-product accumulation within the lower / adjacent structure.

• LoadoutStationSample1-REF-011 is one of the strongest severity images and directly shows major coarse / lump-product accumulation retained on and within surrounding structural members.

• The photographs directly prove distributed spillage / accumulation. Attribution of every deposit specifically to belt carryback remains a mechanism assessment rather than something each still photograph independently demonstrates.

• Preferred first-draft photographs: LoadoutStationSample1-REF-011, REF-008, REF-005 or -006, and REF-007 or -010.

35.4 Batch E Comparison Outcome

• No Batch E visual evidence materially contradicts the current contextual narrative.

• The batch strengthens the contrast between the comparatively clean L2 inspected condition, the localised L4 spillage condition and the extensive / widespread Sample Conveyor 1 condition.

• Sample Conveyor 1 wet-fines wording should remain qualified: adhered / dried deposits support a sticky / moist-material history, while active wet fines are not independently demonstrated by the supplied still set.

• Detailed item-by-item classifications and candidate captions are recorded in Report-Working/evidence_visual_review_batch_E.md.

36. Context Status After Batch E Visual Review

Repository reconciliation and visual verification of Batches A through F are complete. All 23 evidence groups have now been independently reviewed, and the evidence index has been final-consolidated with first-draft figure candidates. The next task is adaptation of the existing report template and preparation of the first populated combined-product spillage report draft.

37. Visual Evidence Review - Batch F

The sixth and final independent visual-evidence review was completed using the uploaded Underground Crusher, UC05-UC01 continuous conveyor system, U3, U5A and U5B photographs. The review compared the media against the existing contextual narrative and retained the distinction between direct physical evidence, source-recorded inspection observations and inferred operating mechanisms.

37.1 Underground Crusher - Visual Verification

• UGCrusher-REF-001 directly shows a sizeable mixed lump/fines accumulation adjacent to the crusher/conveyor area together with local standing water/puddling. The water source is not established by the photograph.

• UGCrusher-REF-002 and -003 directly show mixed coarse fragments and fines accumulated beneath the conveyor/underside structure.

• UGCrusher-REF-005 shows product/fines retained on or behind upper structural members within the guarded area, supporting distributed contamination beyond floor level.

• The existing trace-to-moderate asset-level classification remains appropriate: some locations are locally significant, but the photographic set does not show uniformly widespread accumulation through the full installation.

• Fine material beneath/adjacent to the belt is consistent with the recorded carryback mechanism, but the still photographs do not independently prove the origin of every deposit.

• Preferred first-draft evidence: UGCrusher-REF-001 and UGCrusher-REF-002 or -003.

37.2 UC05 / UC04 / UC03 / UC02 / UC01 Continuous System - Visual Verification

• The 16-photograph set strongly verifies distributed dry-fines accumulation along the continuous conveyor route, with mixed coarse fragments in some locations.

• UGUC05_04_03_02_01-REF-006 and -008 directly show very large local fines mounds beneath/adjacent to the conveyor. REF-015 and -016 provide strong wider evidence of severe local accumulation along the route.

• REF-005 directly shows mixed coarse fragments and fines beneath the conveyor, while several other views are predominantly fine-material deposits.

• REF-007 directly shows a comparatively clear conveyor-side access route. The explanation that this part of the UC05-UC03 area had been cleared shortly before inspection remains source-recorded information; the photograph by itself does not prove the timing/reason for cleaning.

• The route contains both heavily accumulated and comparatively clear sections. The existing moderate, distributed asset-level classification remains appropriate despite some locally large mounds.

• The geometry of deposits beneath/adjacent to the belt is consistent with carryback redistribution, but static photographs do not prove carryback as the sole source of every deposit.

• Preferred first-draft evidence: REF-006 or -008, REF-015 or -016, REF-005, and optional REF-007 as a source-qualified clean/recently-cleared comparator.

37.3 U3 Conveyor - Visual Verification

• UGU3-REF-004, -005 and -007 directly show local fines/mixed-material accumulation immediately beneath and adjacent to conveyor structures and equipment.

• UGU3-REF-002, -003, -006 and -008 show coarse broken rock/material along the underground roadway beside the conveyor. Because the roadway itself is an excavated underground environment, those photographs do not independently establish that every coarse fragment originated from conveyor spillage.

• The existing localised-to-moderate classification remains appropriate. No Batch F photograph resolves the previously Unknown trip/restart clearing condition.

• Preferred first-draft evidence: UGU3-REF-007 plus UGU3-REF-004 or -005; wider roadway views should be used only with the origin qualification.

37.4 U5A Conveyor - Visual Verification

• U5A extensive/widespread dry-fines accumulation is strongly visually verified. Multiple photographs show large fines mounds beneath and alongside the conveyor through the confined route.

• UG5A-REF-002 is a strong large-pile severity image. UG5A-REF-006 and -007 show longitudinal/widespread accumulation along the conveyor side.

• UG5A-REF-004 shows fines immediately beneath the return-belt/roller region, strongly consistent with return-side carryback geometry; however, a still image cannot establish carryback as the sole source of all surrounding fines.

• Preferred first-draft evidence: UG5A-REF-002, UG5A-REF-006 or -007, and UG5A-REF-004.

37.5 U5B Conveyor - Visual Verification

• U5B dry-fines accumulation is directly verified at several local points. UG5B-REF-001 and -005 show sizeable local deposits, while the wider set does not show the same continuous/widespread longitudinal accumulation observed at U5A.

• The existing trace-to-localised overall classification is retained, with the refinement that some individual local deposits are substantial.

• UG5B-REF-003 and -004 show fines immediately beneath the return-belt/support region, strongly consistent with the recorded carryback mechanism; the static images do not prove the source of every deposit.

• Preferred first-draft evidence: UG5B-REF-001 or -005, with UG5B-REF-003 or -004 as optional return-side geometry context.

37.6 Batch F Comparison Outcome

• No Batch F visual evidence materially contradicts the current contextual narrative.

• The final batch strengthens the Crusher, UC-system, U3, U5A and U5B physical-condition records while refining which aspects can be stated as directly visible versus mechanism/source-recorded.

• The strongest new qualifications are: do not automatically attribute all U3 roadway rock to conveyor spillage; keep UC recent-cleaning history source-attributed; retain U5B as trace/localised overall despite sizeable local piles; and do not assign a source to the Crusher standing water without further evidence.

• Detailed item-by-item classifications and candidate captions are recorded in Report-Working/evidence_visual_review_batch_F.md.

38. Context Status After Completion of All Visual Evidence Reviews

All six visual-evidence batches are complete. The full repository evidence population - 23 evidence groups comprising 197 media files - has now been reconciled and independently reviewed. No material visual contradiction has been identified against the consolidated Questionnaire V1 / V2 engineering narrative; where the media cannot prove the full mechanism, the contextual notes now explicitly preserve that qualification.

Report-Working/evidence_index.md has now been final-consolidated with completed review status and initial anchor-image selections. A first evidence-populated report baseline (V0.1) was subsequently created and reviewed. That review confirmed that the underlying engineering content was substantially sound, but that a conventional sheave-style asset-by-asset defect structure was too clinical for this whole-system audit. The project therefore completed a context-alignment and narrative-architecture phase before further report rebuilding. Sections 39 through 43 record the accepted engineering message, hybrid report identity, reader journey, evidence-integration philosophy and controlled next steps.

39. Context Alignment Outcome and Report Identity

Following completion of the six visual-evidence review batches and preparation of the first evidence-populated report baseline, the audit context was deliberately re-examined before further report development. The inspector and report compiler compared the V3.7 engineering model, the V0.1 report experience and the intended message to South32. The factual and mechanism-based interpretation developed in V3.7 was confirmed as substantially aligned. No material engineering theme was identified as missing at the system level. The principal issue exposed by V0.1 was therefore not the technical content, but the report form, writing style and narrative hierarchy used to communicate it.

The final report is to be treated as a hybrid engineering audit: a formally controlled engineering document that uses a guided narrative to explain the whole-system picture, while retaining detailed photographic and technical evidence as the supporting backbone. The report must remain technically defensible and evidence-traceable, but it should not read as a repetitive catalogue of isolated conveyor defects.

39.1 Governing Report Identity

• Formal shell: retain controlled engineering-report components such as the cover page, report metadata, revision history, distribution list, prepared/reviewed/approved sign-off register, contents, limitations, references, final sign-off and controlled appendices.

• Narrative core: guide the reader through the audit story - why the audit was undertaken, how material moves through the system, how containment is being lost, which mechanisms are systemic versus local, why the evidence matters and what control philosophy follows from the combined observations.

• Technical evidence backbone: retain canonical evidence IDs, photographic/video records, detailed case studies, asset-level qualifications, measurements still required, technical actions and supporting appendices.

• The report is therefore an audit narrative first and a technical evidence record second. The evidence proves and refines the story; it should not force the reader through a flat asset-by-asset defect register.

39.2 Intended Reader Experience

The report should be useful to both management/operations and engineering/maintenance readers. A management or operational reader should be able to understand the systemic problem, the major local exceptions, the overall control philosophy and the implementation priorities without reading every detailed evidence record. An engineering or maintenance reader should be able to follow the evidence references, detailed case studies, qualifications and appendices to determine exactly what was observed, what remains inferred or unverified, and what physical inspection, measurement or modification is required.

The preferred writing style is formal engineering narrative. This does not mean informal or less technical language. It means explaining the engineering logic before presenting detailed evidence, using concise narrative paragraphs and bullets where they improve comprehension, and making cause-and-effect relationships explicit through wording such as "This matters because...", "Taken together..." and "The significance of this condition is...".

40. Unified Material-Containment Philosophy

The higher-order engineering conclusion from the audit is broader than belt-cleaner performance or fines control alone. The conveyor ecosystem is intended to transport combined product - fines together with coarse/lump ore - through successive handling, transfer and processing stages while keeping that material within the designed process path. The primary system objective is therefore:

Keep the combined product - fines and coarse/lump ore - within the intended process/material-handling envelope throughout transit, using the control mechanism appropriate to the failure mode and local geometry.

Process water is subject to the same containment principle within the screening/washing system. The report should therefore frame the observed conditions as failures or weaknesses in material/process containment rather than simply as untidy floors or unrelated scraper defects.

40.1 Fines / Carryback and Cleaner-Containment Philosophy

Fines are an inherent part of the combined-product stream. They are present following blasting and additional fines are generated during primary crushing and subsequent handling. The presence of fines is therefore not itself a defect. The problem arises when the fine fraction progressively leaves the intended process stream as carryback or uncontrolled wet/dry spillage before reaching the stage designed to handle it in a controlled manner.

• A belt-cleaning arrangement performs the required material-control function only when residual material is removed from the return belt and the removed material is retained or redirected within the intended process path.

• CV06 remains the principal mechanical/maintenance failure example: displaced support hardware and missing scraper blades prevent the cleaner from performing its intended function. The exact failure history remains unverified.

• CV09 remains the principal cleaner/containment case: wet carryback is removed from the belt, but the removed material can fall outside the effective discharge containment. This demonstrates that belt cleaning and material containment must be treated as one engineering function.

• CV09 is the preferred pilot location for development and testing of an improved cleaner/containment philosophy because the failure mechanism is directly demonstrated and the intended fines-discharge path is identifiable.

• The pilot philosophy should include positive/self-adjusting cleaner contact through wear, practical wear/travel limits, effective containment of removed fines, routing into the intended CV09 fines stream, maintainability and evaluation of primary/secondary cleaning where justified.

• If the pilot is successful, the functional control philosophy should be assessed for use at other applicable conveyor discharges after local geometry, material condition, access and applicability checks. The recommendation is to standardise the required function, not to install identical hardware everywhere. U4 remains an explicit geometry-specific exception requiring separate assessment.

40.2 Coarse / Combined-Product Transit Containment

The audit also identified significant coarse/lump and mixed-product loss from the carrying side of conveyors. These conditions are not explained by return-side carryback and require a separate but compatible containment philosophy. The carrying belt should retain the product within its usable transport envelope until the intended transfer/discharge point.

• Potential coarse-product loss mechanisms include mistracking, loading/trajectory, limited usable belt width, transfer/chute geometry, insufficient lateral containment and event-driven operating conditions. The applicable control must be selected according to the verified mechanism at each location.

• W10 is a key case because significant mixed/coarse product has escaped into a confined tunnel with restricted access. The exact initiating cause of every deposit remains unresolved, but the breach of the carrying envelope and the resulting access/housekeeping consequence are directly established.

• A location-specific Conveyor Side-Containment Barrier / Side-Containment Rail / Side-Containment System may therefore be investigated at W10 as a secondary remedial control. The intended function is to help keep displaced product within the usable belt-carrying zone - effectively within its intended transport lane - without becoming a normal belt-contact surface.

• Any W10 side-containment concept remains subject to verification of belt width, normal tracking envelope, trough geometry, structure, clearances, maintenance access, loading/transfer conditions and trapping/wear risks. It is not a universal instruction to install rails along every conveyor.

• For Sample Conveyors 2 and 3, the control hierarchy remains: first understand/control feed and product loading relative to usable belt width; second verify/correct tracking and tension; third apply secondary side-skirt/containment where appropriate. Sample Conveyor 2 mistracking remains unconfirmed; Sample Conveyor 3 mistracking is visually supported but unmeasured.

40.3 Water and Event-Driven Containment

• Screen-area process-water loss of containment is directly verified, but the exact equipment-level escape mechanism remains unresolved and requires safe shutdown inspection before final modification design.

• Water can mobilise fines and transport contamination away from the initiating point; therefore, visible wet/slurry accumulation does not necessarily identify the exact leak source.

• Trip/restart clearing remains a separate event-driven mechanism. Where material must be temporarily removed from a loaded belt, the control requirement is a defined, contained and recoverable unloading/laydown/reintroduction process rather than a purely mechanical conveyor modification.

• Housekeeping remains necessary to restore access and operating condition, but repeated housekeeping is consequence control. Source control should reduce recurrence.

40.4 Systemic and Local Mechanism Hierarchy

The audit should not imply that all observed spillage is caused by one defect. The mine has a connected set of material-containment weaknesses with one common engineering objective but several different physical solutions.

• Systemic / recurring theme: fines carryback, variable cleaner condition/effectiveness, incomplete containment of removed fines at some discharges and the resulting recurring housekeeping burden.

• Key local / geometry-specific mechanisms: W10 coarse-product transit loss in a confined tunnel; Sample Conveyor feed/width/containment conditions; Sample Conveyor 3 mistracking; U4 alignment/mistracking; screen water escape and support deterioration; L1 historical belt contact and structural deterioration; event-driven trip/restart unloading.

• The point at which material is observed remains distinct from the point at which the initiating mechanism may originate. Carryback, water transport, mistracking and operational unloading can all move the visible consequence away from the original cause location.

41. Accepted Hybrid Report Narrative Architecture

The reader-facing outline recorded in Report-Working/SpillageReport/REPORT_NARRATIVE_OUTLINE_V0.1.md has been reviewed by the inspector and accepted as the baseline narrative map for development of the next report revision. No material engineering theme is presently identified as missing. Minor later additions, changes in emphasis and recommendation expansion are expected and can be incorporated without reopening the fundamental report architecture.

41.1 Formal Front Matter

• Clean professional cover page, retaining the successful visual character of the ISS report lineage.

• Report metadata: report number, revision, issue date, issue status, client/site/project details.

• Document control / revision history, distribution list and prepared/reviewed/approved sign-off register.

• Table of contents, List of Figures and List of Tables in the final controlled issue.

• Formal scope, methodology/evidence basis, limitations/qualifications, terminology and references where required.

41.2 Accepted Reader Journey

• Why the audit was undertaken - establish that the purpose is to understand causes/mechanisms and define practical source controls, not simply catalogue spillage locations.

• Understand the material-handling system first - explain the journey of combined product from the underground crusher through the inspected conveyor ecosystem, screening and loading station.

• Establish the central principle that deposit location does not necessarily equal cause location.

• Explain the overall spillage picture: fines/carryback, coarse/mixed product transit loss, water/wet fines/slurry and event-driven trip/restart unloading.

• Build the systemic fines/carryback story and use CV06/CV09 as complementary case studies that explain cleaner condition versus cleaner-containment function.

• Build the coarse-product story using W10 and the Sample Conveyors, then explain tracking/alignment as a material-containment requirement using U4 and L3 as differentiated examples.

• Explain water/wet-fines and structural consequences using Screens and L1, while preserving the distinction between directly observed condition and unresolved mechanism/capacity.

• Separate operational/event-driven spillage and housekeeping from chronic mechanical/process containment mechanisms.

• Bring the evidence together into one consolidated engineering interpretation and state the material-containment control philosophy before presenting the prioritised action programme.

• Close with a conclusion focused on source control and process containment rather than a cleaner floor alone, followed by the detailed technical evidence backbone in appendices.

41.3 Narrative Writing Rules

• Explain first, prove second. Introduce what the condition means before asking the reader to interpret detailed photographs/tables.

• Use evidence as case proof, not as the report organising principle.

• Do not give every asset equal narrative weight. Some assets are systemic case studies, some are local exceptions and some are supporting/comparator evidence.

• Retain formal evidence IDs and qualifications. Narrative style must not reduce technical defensibility.

• Preserve uncertainty openly. Do not promote an engineering inference, source-recorded observation or working hypothesis into a visually proven fact.

• Use concise paragraphs, bullets and deliberate transitions where they improve comprehension. The target tone is formal engineering narrative, not informal storytelling.

42. Evidence Integration and Layout Principles

The 197 repository media files remain a curated evidence set selected from more than 700 original media items. They should not be treated as a raw pool from which only a small minority is automatically retained. The first-draft selections in Report-Working/evidence_index.md are anchor images only. Final inclusion must consider both evidentiary strength and the role each image plays in the audit story.

42.1 Planned Inclusion Classification

• E - Essential body evidence: required to prove a principal finding, mechanism or narrative point.

• S - Supporting / story evidence: adds unique topology, sequence, extent, operating context, comparison, consequence or design/remedial context and should normally remain in the body where it materially improves understanding.

• A - Appendix / reference evidence: useful record evidence that does not need to interrupt the main narrative.

• X - Exclude from report: genuinely duplicative, weak, blurred or non-contributory. The repository original remains retained.

42.2 Story-Role Classification

• TOPOLOGY / ORIENTATION

• PROCESS RELATIONSHIP

• MECHANISM

• DEFECT / CONDITION

• EXTENT / SEVERITY

• CONSEQUENCE

• COMPARATOR / CLEAN CONDITION

• OPERATING CONTEXT

• MAINTENANCE / DESIGN DETAIL

• REMEDIAL-CONCEPT CONTEXT

42.3 Photographic Evidence Layout Direction

The current V0.1 one-photo-per-page approach is not the intended final evidence architecture. A layout experiment demonstrated that much of the apparent report length was caused by forced pagination rather than unavoidable evidence volume. The future layout should therefore reduce redundant whitespace and repeated metadata rather than discard evidence that contributes a unique technical point.

• Use one technical story/finding per evidence sheet or short evidence sequence rather than one photograph per page.

• Allow one full-width hero image with one or more supporting/context images, two portrait images side-by-side, stacked landscape images or 2 x 2 context/sequence grids where detail remains legible.

• Retain canonical evidence ID and caption for every included image.

• Use shared finding/story-level metadata once where several photographs belong to the same technical story, reducing unnecessary repetition.

• Retain the useful Observation / Engineering Assessment / Recommended Action discipline where it supports a technical story, but do not force the whole report into repetitive evidence-sheet prose.

• Use topology/process context selectively where geometry or process position materially affects interpretation, including W10, Screens, L1, the Sample Conveyors, UC05-UC01, U4 and U5A/U5B.

43. Controlled Context Status After Narrative Alignment

The context-alignment debate is concluded at the baseline level. The inspector has accepted the V3.7 factual/mechanism interpretation, the unified material-containment philosophy and the reader-facing narrative outline as a suitable governing direction for the report. The accepted architecture is intentionally flexible: later evidence selection, measurements, design development or recommendation refinement may require minor additions or changes in emphasis without invalidating the overall narrative structure.

This V3.8 revision therefore becomes the consolidated engineering and narrative context for the next development stages. It preserves the detailed Questionnaire V1/V2 findings and all six visual-review batches from V3.7, while adding the accepted whole-system message, report identity, reader journey and evidence/layout principles developed during the report review and context-alignment phase.

The frozen South32_Spillage_Audit_Report_July2026_S32WPPSR_01_V0.1 remains a historical first evidence-populated baseline and should not be patched incrementally into the final architecture. The next controlled development step is to use this V3.8 context and the accepted narrative outline to classify the full curated evidence set by inclusion status and story role, then prototype and approve the revised photographic evidence module before rebuilding the reader-facing report.

All existing engineering qualification rules remain binding unless later measurements, inspections or additional verified information explicitly resolve them. The report must continue to distinguish direct observation, visually supported condition, source-recorded operating/maintenance information, engineering inference/working hypothesis, remedial concept and unresolved verification requirement.

44. Phase 4 Evidence Classification Closure

The full curated evidence set has now completed dedicated Essential (E), Supporting / Story (S) and Appendix / Reference (A) review passes. The evidence matrix is sufficiently locked for photographic evidence-module prototyping.

Validated final matrix checkpoint:

• Matrix: Report-Working/SpillageReport/EVIDENCE_STORY_MATRIX_V0.1.csv

• Matrix content SHA: 8cb23c01ae59cbc12da9681e95ad70c6b81123df

• Total controlled media: 197

• E - Essential body evidence: 75

• S - Supporting / story evidence: 44

• A - Appendix / reference evidence: 78

• X - Exclude: 0

All 197 matrix rows are inspector-reviewed and marked ACCEPTED. The final E + S story-capable pool contains 119 items. This is a selection pool, not a requirement that all 119 items appear in the body of the report. The 78 A items remain technically valid controlled evidence for appendix/reference use, alternate views, traceability, later design development and layout choice.

No X exclusions were forced. This is deliberate because the 197-item set had already been curated before matrix compilation. The evidence-selection phase has therefore moved from deciding whether an image is technically valid to deciding how the strongest evidence should be grouped and prioritised to tell each engineering story clearly.

45. Phase 5 Prototype Selection - CV09

CV09 has been selected as the first photographic evidence-module prototype. The selection is intentional: CV09 is complex enough to test the report architecture but sufficiently contained to allow careful iteration without introducing the multiple unrelated structural sub-stories present at cases such as Screens or L1.

CV09 is suitable as the first prototype because it contains:

• still photographic evidence;

• dynamic video evidence;

• a visible wet-fines/slurry consequence;

• a meaningful cleaner/chute geometry relationship;

• a clear containment-boundary issue;

• a strong connection to the report-wide fines-control philosophy; and

• a working pilot recommendation that must remain clearly separated from final design.

The prototype is therefore expected to test both technical evidence discipline and reader-facing narrative form.

46. CV09 Controlled Engineering Context

46.1 Core Finding

CV09 demonstrates that belt cleaning and containment of the removed material must be treated as one engineering function. The existing cleaner removes wet residual material from the return belt, but the cleaner/discharge arrangement is positioned such that the removed material is not inherently retained within the effective discharge/process containment envelope. The result is that fines may be removed from the belt while still escaping into the surrounding plant environment.

The functional problem is therefore not simply inadequate belt cleaning. It is inadequate retention of the removed material within the intended process route.

Do not merely clean the belt. Keep the fines inside the process.

46.2 Directly Supported Evidence Position

The accepted evidence supports that:

• wet fines/slurry are present below and around the CV09 cleaner/discharge region;

• the local cleaner/discharge arrangement is open/non-contained relative to the effective process containment boundary;

• active wet-material fallout occurred during operation; and

• the cleaner/chute relationship allows removed material to escape into the surrounding plant area rather than remaining inherently captured within the intended downstream fines-handling route.

46.3 Evidence Hierarchy

E - Essential body evidence:

• CV09-REF-002 - principal wet-fines/slurry consequence.

• CV09-REF-003 - principal cleaner condition/open arrangement still.

• CV09-REF-006 - direct dynamic evidence of active fallout during operation.

• CV09-REF-008 - essential geometry/detail video supporting the cleaner/chute-containment relationship and pilot concept.

S - Supporting / story evidence:

• CV09-REF-005 - wider extent/context below the head/cleaner region.

A - Appendix / reference evidence:

• CV09-REF-001

• CV09-REF-004

• CV09-REF-007

The A items remain controlled evidence but should not occupy prototype body space unless a later layout test identifies a genuinely unique need.

46.4 Qualification Boundary

The current evidence does not establish:

• final chute-extension dimensions;

• final cleaner type;

• quantified scraper contact pressure;

• final wear-compensation arrangement;

• final mounting/support design;

• final access and maintainability geometry; or

• a completed pilot design.

These remain engineering/design-development requirements. The prototype must also avoid implying that every particle visible below CV09 can be assigned to one single instantaneous mechanism.

47. CV09 Prototype Narrative Intent

The CV09 module should not read as a sequence of isolated defect records. It should guide the reader through the following logic:

• Observed condition - substantial wet-fines/slurry contamination is present below the cleaner/discharge region.

• Local mechanism - residual material is being removed from the belt but is not adequately retained within the effective discharge/process containment boundary.

• Operating confirmation - video evidence shows active wet-material fallout during operation.

• Engineering significance - a cleaner that removes material from the belt but allows that material to fall outside the intended process path has only performed part of the required control function.

• Control philosophy - belt cleaning and containment of the removed fines must be integrated.

• Action - CV09 should be used as a pilot/design-development location to develop and trial that functional arrangement.

• System application - if the pilot is successful, the functional philosophy may be assessed for location-specific use at other suitable discharges; identical hardware is not to be prescribed universally.

The module must retain the report's accepted voice:

Explain first, prove second.

The evidence should support the narrative rather than force the narrative into a photograph-by-photograph rhythm.

48. CV09 Working Engineering Assessment and Recommendation Context

48.1 Engineering Assessment Baseline

The intended controlled meaning is:

CV09 demonstrates that belt cleaning and material containment cannot be treated as separate functions. The existing arrangement removes wet residual material from the belt, but removed material is not inherently retained within the effective discharge/process containment envelope. The inspection evidence includes active wet-material fallout during operation and substantial wet-fines/slurry accumulation below the cleaner region. The evidence therefore supports development of an integrated cleaner-and-containment arrangement that removes residual material while directing it back into the intended fines-handling route.

The assessment must retain the qualification that final cleaner type, contact pressure, chute dimensions, support details and maintenance-access geometry require design verification.

48.2 Recommendation Baseline

The intended controlled recommendation is:

Develop and trial a CV09 cleaner/containment arrangement that maintains effective belt-cleaning contact and directs removed wet fines back into the intended chute/process stream. Verify the existing belt/discharge geometry, available containment envelope, cleaner mounting, wear compensation, access, maintainability, safety and downstream fines route before finalising the design. Following successful trial and performance verification, assess the functional cleaner/containment philosophy for location-specific application at other suitable conveyor discharges.

The report must make clear that the recommendation is to standardise the required function, not identical hardware. U4 remains an explicit example where materially different local geometry requires separate assessment.

49. Prototype Evidence and Layout Principles

The CV09 prototype is to establish the reusable evidence-module grammar for Phase 6. The working target is a two-page module, with flexibility to extend to three pages if required for legibility or technical clarity.

The prototype should test:

• narrative-first introduction;

• hero/support image hierarchy;

• efficient multi-photo layout;

• canonical evidence IDs below every figure;

• representative frames from video evidence without inventing new inspection-evidence IDs;

• grouped story-level metadata rather than repetitive per-image metadata;

• concise Observation / Engineering Assessment / Control Philosophy / Recommended Action presentation;

• visible but readable qualifications;

• deliberate page-break control; and

• formal engineering-report appearance without reverting to a repetitive defect-sheet format.

Where a representative frame is extracted from a controlled video, the caption must identify the frame as originating from that video and the evidence register must retain the original .mp4 source.

Example caption form:

CV09-REF-006 - representative frame from inspection video showing active wet-material fallout during operation.

CV09-REF-005 should be used only if the wider context materially improves the story. S classification means the item is available to strengthen the narrative; it does not create an obligation to include it.

50. Controlled Context Status - Phase 5 Active

Phase 4 evidence classification is complete and the CV09 prototype case selection is accepted. The CV09 evidence hierarchy is locked sufficiently for prototyping. The detailed Phase 5 execution plan is maintained at Report-Working/SpillageReport/PHASE5_CV09_PROTOTYPE_EXECUTION_PLAN.md.

Current controlled status:

• Phase 4 evidence classification - COMPLETE.

• CV09 prototype case selection - ACCEPTED.

• CV09 evidence hierarchy - LOCKED SUFFICIENTLY FOR PROTOTYPING.

• Large-scale V0.1 report rebuild - ON HOLD.

• Next action - acquire the CV09 media, select representative video frames, draft the CV09 narrative and build the standalone prototype module.

The frozen South32_Spillage_Audit_Report_July2026_S32WPPSR_01_V0.1 remains unchanged. It is not to be materially revised until the CV09 prototype establishes an accepted evidence density, caption system, visual hierarchy and narrative/evidence interaction.

This V3.9 revision becomes the consolidated governing engineering and narrative context for Phase 5. It preserves all V3.8 content and appends the accepted Phase 4 closure, CV09 prototype selection, controlled CV09 evidentiary meaning and prototype layout/qualification rules.