# Introduction

Combined product is conveyed through a connected mine-to-surface handling chain that includes the underground crusher and UC conveyor route, secondary plant conveyors, silos/feeders, surface screening conveyors, screens/chutes, loading-station conveyors and narrow sample conveyors. Fines are present from blasting and are further generated during crushing and material handling. Water is also introduced in parts of the process, particularly screening/washing.

The inspection found that the visible location of spilled material is not always the location at which the initiating defect occurs. For example, residual material passing the head discharge can remain on the return belt, be detached later by return rollers and appear as deposits some distance downstream. The report therefore evaluates the relationship between observed spillage, immediate mechanism, contributing condition, probable root cause, consequence and proposed action

Purpose

This report documents a qualitative engineering inspection and mechanism-focused assessment of combined-product spillage across selected surface and underground product-handling areas at South32 Wessels Mine. The report is intended to identify where spillage occurs, distinguish the principal physical mechanisms, separate confirmed observations from engineering interpretation, and define practical maintenance, operational and engineering actions for further verification and remedial development

Note : I want to introduce this into the report, preferably near the beginning of the report as there is some terminology that is used in the report that is localized to the report and these terms are not always contextual clear to the user. Suggested location under the Controlled Distribution section before the table of contents?

Definitions and abbreviations

| Term | Definition |
| --- | --- |
| Combined Product | A mixture of lump ore and fines conveyed together as a single material stream. |
| Fines | Small or finely divided ore particles forming the finer fraction of the conveyed material, including very fine particulate material or ore dust. |
| Belt Tracking | The lateral alignment of the conveyor belt along its intended travel path. Correct tracking occurs when the belt remains centred across the conveyor structure and does not exhibit excessive lateral drift. |
| Belt Mistracking | Lateral belt displacement away from the intended centred travel path. |
| Belt Carryback | Residual fines/product adhering to the belt after discharge and returning on the belt rather than being fully removed at the head/discharge point. |
| Belt Scraper / Cleaner | A device installed at or near the discharge to remove material adhering to the belt and, ideally, direct that material into the intended process/discharge path. |
| Wet Fines / Slurry | Moisture-laden fines or slurry-like material produced where fines combine with process or cleaning water. |
| Conveyor Side Skirt / Side-Containment Skirting System | A material-containment arrangement along the conveyor side. Current working concept for the sample conveyors is a fixed side support with flexible rubber skirting close to, but normally not contacting, the belt. |
| Direct visual evidence | A photograph/video that independently demonstrates the stated physical condition. |
| Supporting evidence | Evidence that supports context/extent but does not independently prove the complete causal statement. |

Notes:

- Swap the order of the Purpose and executive summary currently the execution summary is place as