## 9.1 CV06 — Mechanical / Maintenance Failure Example

At CV06 the cleaner cannot perform its intended function because the physical cleaner condition is compromised. The support structure remains installed, but scraper blade elements are absent across the working belt width and a support bush/insert is displaced. Poor housing design was recorded as a probable contributor to the bush displacement, with maintenance/replacement condition also relevant, but the photographs do not establish the root cause or the history of blade loss. The first response is therefore to restore and verify a complete functional cleaner arrangement before considering broader redesign.[for this last sentence lets rephrase/expand by also indicating that it not only needs to be replaced but proposed modifications and upgrades are put forward but discussed in detail later in the recommendations sections etc.]

| CV06-REF-004 — displaced cleaner support bush/insert. |  | CV06-REF-005 — cleaner support structure present with scraper blades absent from the working belt width. |  |
| --- | --- | --- | --- |
| OBSERVATION | Physical cleaner defects are directly visible at CV06. |  |
| ENGINEERING ASSESSMENT | The cleaner is non-functional as intended. Poor housing design was source-recorded as a probable contributor to the displaced bush/insert, but the photographs do not prove the root cause. Blade-loss/removal history also remains unresolved.[Also point out this is an example of where the fines escape and are “spilled” back into the surrounding enviroiment, etc. ] |  |
| RECOMMENDED NEXT ACTION | Restore a complete functional cleaner arrangement; inspect the housing/support condition and retention, reinstate required blade coverage, verify contact/adjustment and maintainability, and confirm representative operating performance before close-out. |  |