# Running Report Contextual Spillage Questionnaire – Response V1

Iteration 1: Spillage Type and Physical Location Identification

Working response document to support the Running Report Contextual Spillage Notes

| Questionnaire Version | V1 |
| --- | --- |
| Contextual Notes Source | Running_Report_Contextual_Spillage_NotesV1 |
| Inspection / Audit |  |
| Completed By |  |
| Date Completed |  |

# 1. Purpose of This Questionnaire

This questionnaire is the first information-gathering iteration for the contextual notes. Its purpose is to establish, for each item within the current inspection scope, what type of spillage was observed and where that spillage was physically located.

At this stage, do not try to fully diagnose the root cause or define the remedial action unless a short note is useful to preserve an observation. Later questionnaire iterations can address mechanisms, contributing conditions, maintenance issues, operating practices, consequences, controls, and recommended actions in greater detail.

# 2. How to Complete the Response

Mark one response in each Yes / No / Unknown / N/A row by placing an X in the appropriate cell.

Where more than one spillage type or physical location applies, mark Yes for every applicable row.

Use Unknown where the condition could not be determined from the inspection or available evidence.

Use N/A where the listed item does not apply to that conveyor, machine, or process location.

Use the notes fields to record short factual observations and photo/video references. Detailed analysis is not required yet.

If no spillage was observed, mark No under 'Any spillage observed'; the remaining tables may then be marked N/A if appropriate.

# 3. Spillage Categories Used in This Iteration

The response options below are derived from the current contextual notes and retain the report's three broad spillage categories: water-related spillage, fines spillage, and solid-product / combined-product spillage. Additional rows distinguish common physical forms already identified in the notes, such as wet fines/slurry, carryback deposits, and material cleared following a trip.

# 4. Surface Processing Plant

NOTE: I have excluded the following conveyor sections CV5, 7, 8, 10, 11, 12, 13. These don not be integrated into the report.

Note: I have add a missing section that was not originally included which is CV15.

## 4.1 Screening Processing Section

### W10 Conveyor — Primary Feed

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed | x |  |  |  |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  | x |  |  |
| Water carrying fines / wet fines / slurry |  | x |  |  |
| Dry fines spillage | x |  |  |  |
| Fines deposited from belt carryback on the return side | x |  |  |  |
| Lump ore / solid-product spillage | x |  |  |  |
| Mixed combined-product spillage (lump ore + fines) | x |  |  |  |
| Product intentionally cleared from a loaded belt following a trip / restart event |  | x |  |  |
| Other material or spillage type (describe below) |  |  |  | x |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point | x |  |  |  |
| Along the carrying side of the conveyor belt | x |  |  |  |
| Along one side of the belt / conveyor structure | x |  |  |  |
| Discharge / transfer point |  |  |  | x |
| Chute / screen / transfer enclosure |  |  |  | x |
| Return side / underside of the conveyor belt | x |  |  |  |
| Directly beneath the conveyor | x |  |  |  |
| Around rollers / idlers / pulleys / other conveyor components | x |  |  |  |
| Walkway / access route | x |  |  |  |
| On surrounding structure / equipment |  |  |  |  |
| Below an elevated conveyor or transfer point |  |  |  | x |
| Other location (describe below) |  |  |  |  |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits | x |  |  |  |
| Localized accumulation | x |  |  |  |
| Moderate accumulation over a defined area | x |  |  |  |
| Extensive / widespread accumulation |  | x |  |  |
| Unable to assess from the inspection | x |  |  |  |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) | Photos references are: W10-REF-001 through to W10-REF-012 |
| Other relevant observation | The Belt Is the primary feed of the combined product into the surface plant from the product that was brought up from underground. It is enclosed tunnel and therefore in a confined space. |

### CV01

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed | x |  |  |  |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  | x |  |  |
| Water carrying fines / wet fines / slurry | x |  |  |  |
| Dry fines spillage | x |  |  |  |
| Fines deposited from belt carryback on the return side | x |  |  |  |
| Lump ore / solid-product spillage |  | x |  |  |
| Mixed combined-product spillage (lump ore + fines) |  | x |  |  |
| Product intentionally cleared from a loaded belt following a trip / restart event |  | x |  |  |
| Other material or spillage type (describe below) | x |  |  |  |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point |  | x |  |  |
| Along the carrying side of the conveyor belt |  | x |  |  |
| Along one side of the belt / conveyor structure | x |  |  |  |
| Discharge / transfer point | x |  |  |  |
| Chute / screen / transfer enclosure | x |  |  |  |
| Return side / underside of the conveyor belt | x |  |  |  |
| Directly beneath the conveyor | x |  |  |  |
| Around rollers / idlers / pulleys / other conveyor components | x |  |  |  |
| Walkway / access route |  | x |  |  |
| On surrounding structure / equipment |  | x |  |  |
| Below an elevated conveyor or transfer point | x |  |  |  |
| Other location (describe below) |  |  |  | x |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits |  | x |  |  |
| Localized accumulation | x |  |  |  |
| Moderate accumulation over a defined area | x |  |  |  |
| Extensive / widespread accumulation |  | x |  |  |
| Unable to assess from the inspection |  |  |  | x |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) | Photo references are CV01-REF-001 through to CV01-REF-004 |
| Other relevant observation | Maintenance problem, return support roller damaged/seized, reference photo is CV01-REF-001. |

### CV02

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed | x |  |  |  |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  | x |  |  |
| Water carrying fines / wet fines / slurry |  | x |  |  |
| Dry fines spillage | x |  |  |  |
| Fines deposited from belt carryback on the return side | x |  |  |  |
| Lump ore / solid-product spillage | x |  |  |  |
| Mixed combined-product spillage (lump ore + fines) | x |  |  |  |
| Product intentionally cleared from a loaded belt following a trip / restart event |  | x |  |  |
| Other material or spillage type (describe below) |  |  |  | x |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point | x |  |  |  |
| Along the carrying side of the conveyor belt |  | x |  |  |
| Along one side of the belt / conveyor structure |  | x |  |  |
| Discharge / transfer point | x |  |  |  |
| Chute / screen / transfer enclosure | x |  |  |  |
| Return side / underside of the conveyor belt | x |  |  |  |
| Directly beneath the conveyor | x |  |  |  |
| Around rollers / idlers / pulleys / other conveyor components |  | x |  |  |
| Walkway / access route | x |  |  |  |
| On surrounding structure / equipment | x |  |  |  |
| Below an elevated conveyor or transfer point | x |  |  |  |
| Other location (describe below) |  |  |  | x |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits | X |  |  |  |
| Localized accumulation | x |  |  |  |
| Moderate accumulation over a defined area | x |  |  |  |
| Extensive / widespread accumulation |  | x |  |  |
| Unable to assess from the inspection |  |  |  | x |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) | Reference photos are CV02-REF-001 through to CV02-REF-004 |
| Other relevant observation |  |

### CV03

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed |  | x |  |  |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  |  |  | X |
| Water carrying fines / wet fines / slurry |  |  |  | X |
| Dry fines spillage |  |  |  | X |
| Fines deposited from belt carryback on the return side |  |  |  | X |
| Lump ore / solid-product spillage |  |  |  | X |
| Mixed combined-product spillage (lump ore + fines) |  |  |  | X |
| Product intentionally cleared from a loaded belt following a trip / restart event |  |  |  | X |
| Other material or spillage type (describe below) |  |  |  | X |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point |  |  |  | X |
| Along the carrying side of the conveyor belt |  |  |  | X |
| Along one side of the belt / conveyor structure |  |  |  | X |
| Discharge / transfer point |  |  |  | X |
| Chute / screen / transfer enclosure |  |  |  | X |
| Return side / underside of the conveyor belt |  |  |  | X |
| Directly beneath the conveyor |  |  |  | X |
| Around rollers / idlers / pulleys / other conveyor components |  |  |  | X |
| Walkway / access route |  |  |  | X |
| On surrounding structure / equipment |  |  |  | X |
| Below an elevated conveyor or transfer point |  |  |  | X |
| Other location (describe below) |  |  |  | x |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits |  |  |  | X |
| Localized accumulation |  |  |  | X |
| Moderate accumulation over a defined area |  |  |  | X |
| Extensive / widespread accumulation |  |  |  | X |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) | Photo References are CV03-REF-001 through to CV03-REF-003 |
| Other relevant observation | none. |

### CV04

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed | X |  |  |  |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  | X |  |  |
| Water carrying fines / wet fines / slurry | X |  |  |  |
| Dry fines spillage | X |  |  |  |
| Fines deposited from belt carryback on the return side | X |  |  |  |
| Lump ore / solid-product spillage |  | X |  |  |
| Mixed combined-product spillage (lump ore + fines) | X |  |  |  |
| Product intentionally cleared from a loaded belt following a trip / restart event |  | X |  |  |
| Other material or spillage type (describe below) |  | X |  |  |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point |  |  | X |  |
| Along the carrying side of the conveyor belt | X |  |  |  |
| Along one side of the belt / conveyor structure | x |  |  |  |
| Discharge / transfer point | X |  |  |  |
| Chute / screen / transfer enclosure | X |  |  |  |
| Return side / underside of the conveyor belt | X |  |  |  |
| Directly beneath the conveyor | X |  |  |  |
| Around rollers / idlers / pulleys / other conveyor components | X |  |  |  |
| Walkway / access route | X |  |  |  |
| On surrounding structure / equipment |  | X |  |  |
| Below an elevated conveyor or transfer point | X |  |  |  |
| Other location (describe below) |  |  |  |  |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits |  | x |  |  |
| Localized accumulation | x |  |  |  |
| Moderate accumulation over a defined area | x |  |  |  |
| Extensive / widespread accumulation | x |  |  |  |
| Unable to assess from the inspection |  |  |  | x |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) | Reference are CV04-REF-001 through CV04-REF-005 |
| Other relevant observation |  |

### CV05

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed |  |  |  | X |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  |  |  | X |
| Water carrying fines / wet fines / slurry |  |  |  | X |
| Dry fines spillage |  |  |  | X |
| Fines deposited from belt carryback on the return side |  |  |  | X |
| Lump ore / solid-product spillage |  |  |  | X |
| Mixed combined-product spillage (lump ore + fines) |  |  |  | X |
| Product intentionally cleared from a loaded belt following a trip / restart event |  |  |  | X |
| Other material or spillage type (describe below) |  |  |  | X |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point |  |  |  | X |
| Along the carrying side of the conveyor belt |  |  |  | X |
| Along one side of the belt / conveyor structure |  |  |  | X |
| Discharge / transfer point |  |  |  | X |
| Chute / screen / transfer enclosure |  |  |  | X |
| Return side / underside of the conveyor belt |  |  |  | X |
| Directly beneath the conveyor |  |  |  | X |
| Around rollers / idlers / pulleys / other conveyor components |  |  |  | X |
| Walkway / access route |  |  |  | X |
| On surrounding structure / equipment |  |  |  | X |
| Below an elevated conveyor or transfer point |  |  |  | X |
| Other location (describe below) |  |  |  | X |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits |  |  |  | X |
| Localized accumulation |  |  |  | X |
| Moderate accumulation over a defined area |  |  |  | X |
| Extensive / widespread accumulation |  |  |  | X |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point | None |
| Photo / video reference(s) |  |
| Other relevant observation |  |

### CV06

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed | X |  |  |  |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  | X |  |  |
| Water carrying fines / wet fines / slurry |  | X |  |  |
| Dry fines spillage | X |  |  |  |
| Fines deposited from belt carryback on the return side | X |  |  |  |
| Lump ore / solid-product spillage |  | X |  |  |
| Mixed combined-product spillage (lump ore + fines) |  | X |  |  |
| Product intentionally cleared from a loaded belt following a trip / restart event |  | X |  |  |
| Other material or spillage type (describe below) |  | X |  |  |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point |  | X |  |  |
| Along the carrying side of the conveyor belt |  | X |  |  |
| Along one side of the belt / conveyor structure |  | X |  |  |
| Discharge / transfer point |  | X |  |  |
| Chute / screen / transfer enclosure |  | X |  |  |
| Return side / underside of the conveyor belt |  | X |  |  |
| Directly beneath the conveyor |  | X |  |  |
| Around rollers / idlers / pulleys / other conveyor components |  | X |  |  |
| Walkway / access route |  | X |  |  |
| On surrounding structure / equipment |  | X |  |  |
| Below an elevated conveyor or transfer point |  | X |  |  |
| Other location (describe below) |  | X |  |  |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits | x |  |  |  |
| Localized accumulation |  | X |  |  |
| Moderate accumulation over a defined area |  | X |  |  |
| Extensive / widespread accumulation |  | X |  |  |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) | Photo References are CV06-REF-001 through CV06-REF-005 |
| Other relevant observation | photo CV06-REF-004 Shows ineffective scrapper support bushing malfunction. The bush displaced from housing. Investigate possible global re-design of conveyor belt scrapper mechanism for chutes. photo CV06-REF-005 Shows head scrapper blades completely missing. |

### CV07

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed |  |  |  | X |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  |  |  | X |
| Water carrying fines / wet fines / slurry |  |  |  | X |
| Dry fines spillage |  |  |  | X |
| Fines deposited from belt carryback on the return side |  |  |  | X |
| Lump ore / solid-product spillage |  |  |  | X |
| Mixed combined-product spillage (lump ore + fines) |  |  |  | X |
| Product intentionally cleared from a loaded belt following a trip / restart event |  |  |  | X |
| Other material or spillage type (describe below) |  |  |  | X |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point |  |  |  | X |
| Along the carrying side of the conveyor belt |  |  |  | X |
| Along one side of the belt / conveyor structure |  |  |  | X |
| Discharge / transfer point |  |  |  | X |
| Chute / screen / transfer enclosure |  |  |  | X |
| Return side / underside of the conveyor belt |  |  |  | X |
| Directly beneath the conveyor |  |  |  | X |
| Around rollers / idlers / pulleys / other conveyor components |  |  |  | X |
| Walkway / access route |  |  |  | X |
| On surrounding structure / equipment |  |  |  | X |
| Below an elevated conveyor or transfer point |  |  |  | X |
| Other location (describe below) |  |  |  | X |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits |  |  |  | X |
| Localized accumulation |  |  |  | X |
| Moderate accumulation over a defined area |  |  |  | X |
| Extensive / widespread accumulation |  |  |  | X |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) |  |
| Other relevant observation |  |

### CV08

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed |  |  |  | X |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  |  |  | X |
| Water carrying fines / wet fines / slurry |  |  |  | X |
| Dry fines spillage |  |  |  | X |
| Fines deposited from belt carryback on the return side |  |  |  | X |
| Lump ore / solid-product spillage |  |  |  | X |
| Mixed combined-product spillage (lump ore + fines) |  |  |  | X |
| Product intentionally cleared from a loaded belt following a trip / restart event |  |  |  | X |
| Other material or spillage type (describe below) |  |  |  | X |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point |  |  |  | X |
| Along the carrying side of the conveyor belt |  |  |  | X |
| Along one side of the belt / conveyor structure |  |  |  | X |
| Discharge / transfer point |  |  |  | X |
| Chute / screen / transfer enclosure |  |  |  | X |
| Return side / underside of the conveyor belt |  |  |  | X |
| Directly beneath the conveyor |  |  |  | X |
| Around rollers / idlers / pulleys / other conveyor components |  |  |  | X |
| Walkway / access route |  |  |  | X |
| On surrounding structure / equipment |  |  |  | X |
| Below an elevated conveyor or transfer point |  |  |  | X |
| Other location (describe below) |  |  |  | X |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits |  |  |  | X |
| Localized accumulation |  |  |  | X |
| Moderate accumulation over a defined area |  |  |  | X |
| Extensive / widespread accumulation |  |  |  | X |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) |  |
| Other relevant observation |  |

### CV09

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed | X |  |  |  |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  | X |  |  |
| Water carrying fines / wet fines / slurry | X |  |  |  |
| Dry fines spillage |  | X |  |  |
| Fines deposited from belt carryback on the return side | X |  |  |  |
| Lump ore / solid-product spillage |  | X |  |  |
| Mixed combined-product spillage (lump ore + fines) |  | X |  |  |
| Product intentionally cleared from a loaded belt following a trip / restart event |  | X |  |  |
| Other material or spillage type (describe below) |  | X |  |  |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point |  | X |  |  |
| Along the carrying side of the conveyor belt | X |  |  |  |
| Along one side of the belt / conveyor structure |  | X |  |  |
| Discharge / transfer point | X |  |  |  |
| Chute / screen / transfer enclosure | X |  |  |  |
| Return side / underside of the conveyor belt | X |  |  |  |
| Directly beneath the conveyor | X |  |  |  |
| Around rollers / idlers / pulleys / other conveyor components | X |  |  |  |
| Walkway / access route |  | X |  |  |
| On surrounding structure / equipment | X |  |  |  |
| Below an elevated conveyor or transfer point | X |  |  |  |
| Other location (describe below) |  | X |  |  |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits |  | X |  |  |
| Localized accumulation |  | X |  |  |
| Moderate accumulation over a defined area |  |  |  |  |
| Extensive / widespread accumulation | x |  |  |  |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) | Photo References are: CV09-REF-001 through CV09-REF-005 Video References are: CV09-REF-006 through CV09-REF-008 |
| Other relevant observation | the scrapper that is employed is of verry poor design. The scrapper is shown in CV09-REF-003 and video CV09-REF-008. The scrapper is on the outside of the discharge chute so even the wet carryback on the underside of the that it catches/scrapes which is also not very effective to start with, falls down below the conveyor into the open environment. Furthermore, the scrapper is not even effective at scrapping/catching the carryback that the support rollers downstream rub off the carryback further spreading the spillage in other areas. Including the main screen house floors below. That is shown in the photos and the videos. The scrapper needs to redesigned to be located inside the discharge chute section even if that means resigning the discharge chute housing to be extended out wards so that the scrapper section is encased inside. So, the scrapper can deposit the excess scrapped of into the discharge chur with the original material discharged. Furthermore, the scrapper needs to be on a spring load design so that it can maintain positive contact pressure with the conveyor belt as it wears down. The spring-loaded scrapper also must have upper limiters that once the wear pads wear to the maximum they are stopped in order to prevent unnecessary wear damage to the conveyor belt. |

### CV10

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed |  |  |  | X |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  |  |  | X |
| Water carrying fines / wet fines / slurry |  |  |  | X |
| Dry fines spillage |  |  |  | X |
| Fines deposited from belt carryback on the return side |  |  |  | X |
| Lump ore / solid-product spillage |  |  |  | X |
| Mixed combined-product spillage (lump ore + fines) |  |  |  | X |
| Product intentionally cleared from a loaded belt following a trip / restart event |  |  |  | X |
| Other material or spillage type (describe below) |  |  |  | X |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point |  |  |  | X |
| Along the carrying side of the conveyor belt |  |  |  | X |
| Along one side of the belt / conveyor structure |  |  |  | X |
| Discharge / transfer point |  |  |  | X |
| Chute / screen / transfer enclosure |  |  |  | X |
| Return side / underside of the conveyor belt |  |  |  | X |
| Directly beneath the conveyor |  |  |  | X |
| Around rollers / idlers / pulleys / other conveyor components |  |  |  | X |
| Walkway / access route |  |  |  | X |
| On surrounding structure / equipment |  |  |  | X |
| Below an elevated conveyor or transfer point |  |  |  | X |
| Other location (describe below) |  |  |  | X |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits |  |  |  | X |
| Localized accumulation |  |  |  | X |
| Moderate accumulation over a defined area |  |  |  | X |
| Extensive / widespread accumulation |  |  |  | X |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) |  |
| Other relevant observation |  |

### CV11

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed |  |  |  | X |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  |  |  | X |
| Water carrying fines / wet fines / slurry |  |  |  | X |
| Dry fines spillage |  |  |  | X |
| Fines deposited from belt carryback on the return side |  |  |  | X |
| Lump ore / solid-product spillage |  |  |  | X |
| Mixed combined-product spillage (lump ore + fines) |  |  |  | X |
| Product intentionally cleared from a loaded belt following a trip / restart event |  |  |  | X |
| Other material or spillage type (describe below) |  |  |  | X |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point |  |  |  | X |
| Along the carrying side of the conveyor belt |  |  |  | X |
| Along one side of the belt / conveyor structure |  |  |  | X |
| Discharge / transfer point |  |  |  | X |
| Chute / screen / transfer enclosure |  |  |  | X |
| Return side / underside of the conveyor belt |  |  |  | X |
| Directly beneath the conveyor |  |  |  | X |
| Around rollers / idlers / pulleys / other conveyor components |  |  |  | X |
| Walkway / access route |  |  |  | X |
| On surrounding structure / equipment |  |  |  | X |
| Below an elevated conveyor or transfer point |  |  |  | X |
| Other location (describe below) |  |  |  | X |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits |  |  |  | X |
| Localized accumulation |  |  |  | X |
| Moderate accumulation over a defined area |  |  |  | X |
| Extensive / widespread accumulation |  |  |  | X |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) |  |
| Other relevant observation |  |

### CV12

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed |  |  |  | X |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  |  |  | X |
| Water carrying fines / wet fines / slurry |  |  |  | X |
| Dry fines spillage |  |  |  | X |
| Fines deposited from belt carryback on the return side |  |  |  | X |
| Lump ore / solid-product spillage |  |  |  | X |
| Mixed combined-product spillage (lump ore + fines) |  |  |  | X |
| Product intentionally cleared from a loaded belt following a trip / restart event |  |  |  | X |
| Other material or spillage type (describe below) |  |  |  | X |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point |  |  |  | X |
| Along the carrying side of the conveyor belt |  |  |  | X |
| Along one side of the belt / conveyor structure |  |  |  | X |
| Discharge / transfer point |  |  |  | X |
| Chute / screen / transfer enclosure |  |  |  | X |
| Return side / underside of the conveyor belt |  |  |  | X |
| Directly beneath the conveyor |  |  |  | X |
| Around rollers / idlers / pulleys / other conveyor components |  |  |  | X |
| Walkway / access route |  |  |  | X |
| On surrounding structure / equipment |  |  |  | X |
| Below an elevated conveyor or transfer point |  |  |  | X |
| Other location (describe below) |  |  |  | X |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits |  |  |  | X |
| Localized accumulation |  |  |  | X |
| Moderate accumulation over a defined area |  |  |  | X |
| Extensive / widespread accumulation |  |  |  | X |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) |  |
| Other relevant observation |  |

### CV13

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed |  |  |  | X |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  |  |  | X |
| Water carrying fines / wet fines / slurry |  |  |  | X |
| Dry fines spillage |  |  |  | X |
| Fines deposited from belt carryback on the return side |  |  |  | X |
| Lump ore / solid-product spillage |  |  |  | X |
| Mixed combined-product spillage (lump ore + fines) |  |  |  | X |
| Product intentionally cleared from a loaded belt following a trip / restart event |  |  |  | X |
| Other material or spillage type (describe below) |  |  |  | X |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point |  |  |  | X |
| Along the carrying side of the conveyor belt |  |  |  | X |
| Along one side of the belt / conveyor structure |  |  |  | X |
| Discharge / transfer point |  |  |  | X |
| Chute / screen / transfer enclosure |  |  |  | X |
| Return side / underside of the conveyor belt |  |  |  | X |
| Directly beneath the conveyor |  |  |  | X |
| Around rollers / idlers / pulleys / other conveyor components |  |  |  | X |
| Walkway / access route |  |  |  | X |
| On surrounding structure / equipment |  |  |  | X |
| Below an elevated conveyor or transfer point |  |  |  | X |
| Other location (describe below) |  |  |  | X |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits |  |  |  | X |
| Localized accumulation |  |  |  | X |
| Moderate accumulation over a defined area |  |  |  | X |
| Extensive / widespread accumulation |  |  |  | X |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) |  |
| Other relevant observation |  |

### CV14

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed | X |  |  |  |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  | X |  |  |
| Water carrying fines / wet fines / slurry |  | X |  |  |
| Dry fines spillage | X |  |  |  |
| Fines deposited from belt carryback on the return side | x |  |  |  |
| Lump ore / solid-product spillage |  | X |  |  |
| Mixed combined-product spillage (lump ore + fines) |  | X |  |  |
| Product intentionally cleared from a loaded belt following a trip / restart event |  | X |  |  |
| Other material or spillage type (describe below) |  | X |  |  |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point |  | X |  |  |
| Along the carrying side of the conveyor belt |  | X |  |  |
| Along one side of the belt / conveyor structure |  | X |  |  |
| Discharge / transfer point | X |  |  |  |
| Chute / screen / transfer enclosure |  | X |  |  |
| Return side / underside of the conveyor belt | X |  |  |  |
| Directly beneath the conveyor | X |  |  |  |
| Around rollers / idlers / pulleys / other conveyor components |  | X |  |  |
| Walkway / access route |  | X |  |  |
| On surrounding structure / equipment |  | X |  |  |
| Below an elevated conveyor or transfer point | X |  |  |  |
| Other location (describe below) |  | X |  |  |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits | X |  |  |  |
| Localized accumulation | X |  |  |  |
| Moderate accumulation over a defined area |  | X |  |  |
| Extensive / widespread accumulation |  | X |  |  |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) | Reference Photos are CV14-REF-001 and CV14-REF-002 |
| Other relevant observation |  |

### CV15

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed | X |  |  |  |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  | X |  |  |
| Water carrying fines / wet fines / slurry | x |  |  |  |
| Dry fines spillage | X |  |  |  |
| Fines deposited from belt carryback on the return side | x |  |  |  |
| Lump ore / solid-product spillage |  | X |  |  |
| Mixed combined-product spillage (lump ore + fines) |  | X |  |  |
| Product intentionally cleared from a loaded belt following a trip / restart event |  | X |  |  |
| Other material or spillage type (describe below) |  | X |  |  |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point |  | X |  |  |
| Along the carrying side of the conveyor belt |  | X |  |  |
| Along one side of the belt / conveyor structure |  | X |  |  |
| Discharge / transfer point | X |  |  |  |
| Chute / screen / transfer enclosure | X |  |  |  |
| Return side / underside of the conveyor belt | X |  |  |  |
| Directly beneath the conveyor | X |  |  |  |
| Around rollers / idlers / pulleys / other conveyor components | X |  |  |  |
| Walkway / access route | X |  |  |  |
| On surrounding structure / equipment | X |  |  |  |
| Below an elevated conveyor or transfer point | X |  |  |  |
| Other location (describe below) |  | X |  |  |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits | X |  |  |  |
| Localized accumulation | X |  |  |  |
| Moderate accumulation over a defined area | X |  |  |  |
| Extensive / widespread accumulation | X |  |  |  |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point | Originating around from chute Number E-3181 |
| Photo / video reference(s) | Reference Photos are CV15-REF-001 and CV15-REF-008 |
| Other relevant observation |  |

### Screening Processing Section — Section Summary

NOTE: I have added one section that was missing  from the CV series belts namely : CV15

Note:  There is an additional section that forms part of the processing plant section and that is the screens that sperate/sort the product by size and sends product that is too big back for further processing/crushing. The screen also washes the product to separate the fines out from the combined product. The problem that was identified at the screens is a failure to contain the water that is used inside the screening process, essentially inside the screen chutes the water was and spills out the side of the screen chute. The result is long term corrosion of the screen support structures and screen building internal floor supports and surrounding areas. Evidence of this is shown in the phots and videos.

Furthermore, the main supports that the screens are mounted are damaged and been purposefully cut. This is not acceptable as they are load bearing supports. This needs to be rectified and scheduled for repair, possible at the next time the screens are changed/replaced the supports need to be replaced as well. It is suspected that the supports were  cut in an attempt to make the installation of the screens easier to install during a replacement cycle. If that is the case the new supports must be repositioned to allow for sufficient clearance to allow for the installation in future rectification plans. A qualified evaluation if the dimension required and repositioning of the new/replacement supports must be conducted to quantify the requirements for the rectification actions required done by a qualified structural authority.  Reference is supplied in the photos.

The reference photos for the screen/screen chutes are :

Photo References :  Screen-REF-001 through Screen-REF-0014

Video References:  Screen-REF-015 through Screen-REF-017

| Common spillage pattern(s) | Fines and more specifically wet Fines that are not effectively controlled/collected, that is transfer around and spread via carryback on the underside of the conveyor belts. Note: A possible solution Fines/carryback, for future rounds the ask me to define describe he possible solutions regarding the modification of the discharge ends/chutes and scrapper modifications/design proposal. |
| --- | --- |
| Assets with the most significant visible accumulation | CV15 |
| Any spillage that appears common to several assets |  |
| General comments / uncertainties |  |

## 4.2 Loading Station Processing Section

### L1 Conveyor — Primary Feed

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed | X |  |  |  |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  | X |  |  |
| Water carrying fines / wet fines / slurry |  | X |  |  |
| Dry fines spillage | X |  |  |  |
| Fines deposited from belt carryback on the return side | X |  |  |  |
| Lump ore / solid-product spillage | X |  |  |  |
| Mixed combined-product spillage (lump ore + fines) | X |  |  |  |
| Product intentionally cleared from a loaded belt following a trip / restart event |  | X |  |  |
| Other material or spillage type (describe below) |  |  |  | X |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point |  | X |  |  |
| Along the carrying side of the conveyor belt |  | X |  |  |
| Along one side of the belt / conveyor structure | X |  |  |  |
| Discharge / transfer point | X |  |  |  |
| Chute / screen / transfer enclosure | X |  |  |  |
| Return side / underside of the conveyor belt | X |  |  |  |
| Directly beneath the conveyor | X |  |  |  |
| Around rollers / idlers / pulleys / other conveyor components | X |  |  |  |
| Walkway / access route | X |  |  |  |
| On surrounding structure / equipment | X |  |  |  |
| Below an elevated conveyor or transfer point | X |  |  |  |
| Other location (describe below) |  |  |  | X |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits | X |  |  |  |
| Localized accumulation | X |  |  |  |
| Moderate accumulation over a defined area | X |  |  |  |
| Extensive / widespread accumulation | X |  |  |  |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) | Photo References are LoadoutStationL1-REF-001 through LoadoutStationL1-REF-018 |
| Other relevant observation | Structural damage from the belt shown in photos: - LoadoutStationL1-REF-007 L1 Tunnel Structural damage shown in photos: - LoadoutStationL1-REF-003, - LoadoutStationL1-REF-004, - LoadoutStationL1-REF-005, - LoadoutStationL1-REF-006, - LoadoutStationL1-REF-007 - LoadoutStationL1-REF-008 Structural corrosion shown in photos: - LoadoutStationL1-REF-008, - LoadoutStationL1-REF-009 |

### L2 Conveyor

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed |  | X |  |  |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  | X |  |  |
| Water carrying fines / wet fines / slurry |  | X |  |  |
| Dry fines spillage |  | X |  |  |
| Fines deposited from belt carryback on the return side |  | X |  |  |
| Lump ore / solid-product spillage |  | X |  |  |
| Mixed combined-product spillage (lump ore + fines) |  | X |  |  |
| Product intentionally cleared from a loaded belt following a trip / restart event |  | X |  |  |
| Other material or spillage type (describe below) |  | X |  |  |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point |  | X |  |  |
| Along the carrying side of the conveyor belt |  | X |  |  |
| Along one side of the belt / conveyor structure |  | X |  |  |
| Discharge / transfer point |  | X |  |  |
| Chute / screen / transfer enclosure |  | X |  |  |
| Return side / underside of the conveyor belt |  | X |  |  |
| Directly beneath the conveyor |  | X |  |  |
| Around rollers / idlers / pulleys / other conveyor components |  | X |  |  |
| Walkway / access route |  | X |  |  |
| On surrounding structure / equipment |  | X |  |  |
| Below an elevated conveyor or transfer point |  | X |  |  |
| Other location (describe below) |  | X |  |  |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits |  | X |  |  |
| Localized accumulation |  | X |  |  |
| Moderate accumulation over a defined area |  | X |  |  |
| Extensive / widespread accumulation |  | X |  |  |
| Unable to assess from the inspection |  | X |  |  |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) | Reference Photos are: LoadoutStationL2-REF-001 through LoadoutStationL2-REF-006 |
| Other relevant observation |  |

### L3 Conveyor

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed | X |  |  |  |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  | X |  |  |
| Water carrying fines / wet fines / slurry |  | X |  |  |
| Dry fines spillage | X |  |  |  |
| Fines deposited from belt carryback on the return side | X |  |  |  |
| Lump ore / solid-product spillage |  | X |  |  |
| Mixed combined-product spillage (lump ore + fines) |  | X |  |  |
| Product intentionally cleared from a loaded belt following a trip / restart event |  | X |  |  |
| Other material or spillage type (describe below) |  | X |  |  |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point | X |  |  |  |
| Along the carrying side of the conveyor belt |  | X |  |  |
| Along one side of the belt / conveyor structure |  | X |  |  |
| Discharge / transfer point | X |  |  |  |
| Chute / screen / transfer enclosure |  | X |  |  |
| Return side / underside of the conveyor belt | X |  |  |  |
| Directly beneath the conveyor | X |  |  |  |
| Around rollers / idlers / pulleys / other conveyor components | X |  |  |  |
| Walkway / access route | X |  |  |  |
| On surrounding structure / equipment |  | X |  |  |
| Below an elevated conveyor or transfer point | X |  |  |  |
| Other location (describe below) |  | X |  |  |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits | X |  |  |  |
| Localized accumulation | X |  |  |  |
| Moderate accumulation over a defined area |  | X |  |  |
| Extensive / widespread accumulation |  | X |  |  |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) | Photo References are : LoadoutStationL3-REF-001 through LoadoutStationL3-REF-008 |
| Other relevant observation | Tail-end Pully conveyor belts off-set shown in LoadoutStationL3-REF-001 |

### L4 Conveyor

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed | X |  |  |  |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  | X |  |  |
| Water carrying fines / wet fines / slurry |  | X |  |  |
| Dry fines spillage | X |  |  |  |
| Fines deposited from belt carryback on the return side |  | X |  |  |
| Lump ore / solid-product spillage | X |  |  |  |
| Mixed combined-product spillage (lump ore + fines) |  | X |  |  |
| Product intentionally cleared from a loaded belt following a trip / restart event |  | X |  |  |
| Other material or spillage type (describe below) |  | X |  |  |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point |  | X |  |  |
| Along the carrying side of the conveyor belt |  | X |  |  |
| Along one side of the belt / conveyor structure | X |  |  |  |
| Discharge / transfer point |  | X |  |  |
| Chute / screen / transfer enclosure |  | X |  |  |
| Return side / underside of the conveyor belt |  | X |  |  |
| Directly beneath the conveyor |  | X |  |  |
| Around rollers / idlers / pulleys / other conveyor components | X |  |  |  |
| Walkway / access route |  | X |  |  |
| On surrounding structure / equipment |  | X |  |  |
| Below an elevated conveyor or transfer point |  | X |  |  |
| Other location (describe below) |  | X |  |  |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits | X |  |  |  |
| Localized accumulation | X |  |  |  |
| Moderate accumulation over a defined area |  | X |  |  |
| Extensive / widespread accumulation |  | X |  |  |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) | Photo Reference are: LoadoutStationL4-REF-001 Through LoadoutStationL4-REF-008 |
| Other relevant observation |  |

### Sample Conveyor 1

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed | X |  |  |  |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  | X |  |  |
| Water carrying fines / wet fines / slurry | X |  |  |  |
| Dry fines spillage | X |  |  |  |
| Fines deposited from belt carryback on the return side | X |  |  |  |
| Lump ore / solid-product spillage | X |  |  |  |
| Mixed combined-product spillage (lump ore + fines) | X |  |  |  |
| Product intentionally cleared from a loaded belt following a trip / restart event |  | X |  |  |
| Other material or spillage type (describe below) |  | X |  |  |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point |  | X |  |  |
| Along the carrying side of the conveyor belt |  | X |  |  |
| Along one side of the belt / conveyor structure | X |  |  |  |
| Discharge / transfer point | X |  |  |  |
| Chute / screen / transfer enclosure | X |  |  |  |
| Return side / underside of the conveyor belt | X |  |  |  |
| Directly beneath the conveyor | X |  |  |  |
| Around rollers / idlers / pulleys / other conveyor components | X |  |  |  |
| Walkway / access route | X |  |  |  |
| On surrounding structure / equipment | X |  |  |  |
| Below an elevated conveyor or transfer point | X |  |  |  |
| Other location (describe below) |  | X |  |  |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits | X |  |  |  |
| Localized accumulation |  | X |  |  |
| Moderate accumulation over a defined area | X |  |  |  |
| Extensive / widespread accumulation | X |  |  |  |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) | Photo Refences are : LoadoutStationSample1-REF-001 through LoadoutStationSample1-REF-011 |
| Other relevant observation |  |

### Sample Conveyor 2

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed | X |  |  |  |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  | X |  |  |
| Water carrying fines / wet fines / slurry |  | X |  |  |
| Dry fines spillage | X |  |  |  |
| Fines deposited from belt carryback on the return side | X |  |  |  |
| Lump ore / solid-product spillage | X |  |  |  |
| Mixed combined-product spillage (lump ore + fines) | X |  |  |  |
| Product intentionally cleared from a loaded belt following a trip / restart event |  | x |  |  |
| Other material or spillage type (describe below) | X |  |  |  |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point |  |  |  |  |
| Along the carrying side of the conveyor belt | X |  |  |  |
| Along one side of the belt / conveyor structure | X |  |  |  |
| Discharge / transfer point |  | X |  |  |
| Chute / screen / transfer enclosure |  | X |  |  |
| Return side / underside of the conveyor belt | X |  |  |  |
| Directly beneath the conveyor | X |  |  |  |
| Around rollers / idlers / pulleys / other conveyor components | X |  |  |  |
| Walkway / access route | X |  |  |  |
| On surrounding structure / equipment | X |  |  |  |
| Below an elevated conveyor or transfer point | X |  |  |  |
| Other location (describe below) |  |  |  | X |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits |  | X |  |  |
| Localized accumulation |  | X |  |  |
| Moderate accumulation over a defined area |  | X |  |  |
| Extensive / widespread accumulation | X |  |  |  |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) | Photo References are: LoadoutStationSample2-REF-001 Through LoadoutStationSample2-REF-008 |
| Other relevant observation | Conveyor belt product load and size to belt size ratio is inefficient, the belt cannot contain or carry product of the size and volume placed on the belt. The entry of the product onto this belt section requires to be throttled/restricted. An evaluation of the mechanisms that control the through put of the sample product that enter the belt(put onto the belt) must be evaluated and possible redesigned to ensure the correct volume of product is allowed to be processed onto the sample belt to be transported to the sample stock pile. NOTE: I have not yet defined what to this proposed solution should be called. But I will describe the solution here and possibly you can help me give the solution a name or define a description: the solution: The possible guard rail be placed/installed along the sides of the conveyor to aid in keeping the product centered on the belt and help prevent the product from falling off the belt. Specific photo reference is: LoadoutStationSample2-REF-004 LoadoutStationSample2-REF-005 LoadoutStationSample2-REF-006 LoadoutStationSample2-REF-007 |

### Sample Conveyor 3

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed | X |  |  |  |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  | X |  |  |
| Water carrying fines / wet fines / slurry |  | X |  |  |
| Dry fines spillage |  | X |  |  |
| Fines deposited from belt carryback on the return side |  | X |  |  |
| Lump ore / solid-product spillage | X |  |  |  |
| Mixed combined-product spillage (lump ore + fines) | X |  |  |  |
| Product intentionally cleared from a loaded belt following a trip / restart event | X |  |  |  |
| Other material or spillage type (describe below) |  |  |  | X |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point | X |  |  |  |
| Along the carrying side of the conveyor belt | X |  |  |  |
| Along one side of the belt / conveyor structure | X |  |  |  |
| Discharge / transfer point |  |  |  |  |
| Chute / screen / transfer enclosure | X |  |  |  |
| Return side / underside of the conveyor belt |  | X |  |  |
| Directly beneath the conveyor | X |  |  |  |
| Around rollers / idlers / pulleys / other conveyor components | X |  |  |  |
| Walkway / access route | X |  |  |  |
| On surrounding structure / equipment | X |  |  |  |
| Below an elevated conveyor or transfer point | X |  |  |  |
| Other location (describe below) |  |  |  | X |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits |  | X |  |  |
| Localized accumulation |  | X |  |  |
| Moderate accumulation over a defined area |  | X |  |  |
| Extensive / widespread accumulation | X |  |  |  |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) | Photo References are: LoadoutStationSample3-REF-001 Through LoadoutStationSample3-REF-013 |
| Other relevant observation | Note this is the same description as in Sample 2 conveyor but same situation but the only additional observation is the belt has Belt Mistracking. Conveyor belt product load and size to belt size ratio is inefficient, the belt cannot contain or carry product of the size and volume placed on the belt. The entry of the product onto this belt section requires to be throttled/restricted. An evaluation of the mechanisms that control the through put of the sample product that enter the belt(put onto the belt) must be evaluated and possible redesigned to ensure the correct volume of product is allowed to be processed onto the sample belt to be transported to the sample stock pile. NOTE: I have not yet defined what to this proposed solution should be called. But I will describe the solution here and possibly you can help me give the solution a name or define a description: the solution: The possible guard rail be placed/installed along the sides of the conveyor to aid in keeping the product centered on the belt and help prevent the product from falling off the belt. Specific photo reference is: LoadoutStationSample3-REF-002 LoadoutStationSample3-REF-003 LoadoutStationSample3-REF-004 LoadoutStationSample3-REF-005 LoadoutStationSample3-REF-006 LoadoutStationSample3-REF-007 |

### Loading Station Processing Section — Section Summary

| Common spillage pattern(s) | Dry fines spillage, Mixed combined-product spillage (lump ore + fines) |
| --- | --- |
| Assets with the most significant visible accumulation | Sample Conveyor 1, Sample Conveyor 2, Sample Conveyor 3. |
| Any spillage that appears common to several assets | Dry fines spillage, Mixed combined-product spillage (lump ore + fines) |
| General comments / uncertainties |  |

# 5. Underground Sections

## 5.1 Central Block

### Crusher

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed | x |  |  |  |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  | x |  |  |
| Water carrying fines / wet fines / slurry |  | x |  |  |
| Dry fines spillage | x |  |  |  |
| Fines deposited from belt carryback on the return side | x |  |  |  |
| Lump ore / solid-product spillage | x |  |  |  |
| Mixed combined-product spillage (lump ore + fines) |  | X |  |  |
| Product intentionally cleared from a loaded belt following a trip / restart event |  | X |  |  |
| Other material or spillage type (describe below) |  |  |  | X |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point |  | x |  |  |
| Along the carrying side of the conveyor belt |  | X |  |  |
| Along one side of the belt / conveyor structure | X |  |  |  |
| Discharge / transfer point |  | x |  |  |
| Chute / screen / transfer enclosure |  | X |  |  |
| Return side / underside of the conveyor belt | X |  |  |  |
| Directly beneath the conveyor | X |  |  |  |
| Around rollers / idlers / pulleys / other conveyor components | X |  |  |  |
| Walkway / access route |  | X |  |  |
| On surrounding structure / equipment |  | X |  |  |
| Below an elevated conveyor or transfer point |  | X |  |  |
| Other location (describe below) |  |  |  | X |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits | X |  |  |  |
| Localized accumulation | X |  |  |  |
| Moderate accumulation over a defined area | X |  |  |  |
| Extensive / widespread accumulation |  | X |  |  |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) | Photo References are: UGCrusher-REF-001 Through UGCrusher-REF-005 |
| Other relevant observation |  |

### UC05 – 04 – 03 – 02 - 01

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed | X |  |  |  |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  | X |  |  |
| Water carrying fines / wet fines / slurry |  | X |  |  |
| Dry fines spillage | X |  |  |  |
| Fines deposited from belt carryback on the return side | X |  |  |  |
| Lump ore / solid-product spillage |  | X |  |  |
| Mixed combined-product spillage (lump ore + fines) | X |  |  |  |
| Product intentionally cleared from a loaded belt following a trip / restart event |  | X |  |  |
| Other material or spillage type (describe below) |  |  |  | X |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point | X |  |  |  |
| Along the carrying side of the conveyor belt |  | X |  |  |
| Along one side of the belt / conveyor structure | X |  |  |  |
| Discharge / transfer point | X |  |  |  |
| Chute / screen / transfer enclosure | X |  |  |  |
| Return side / underside of the conveyor belt | X |  |  |  |
| Directly beneath the conveyor | X |  |  |  |
| Around rollers / idlers / pulleys / other conveyor components |  | X |  |  |
| Walkway / access route |  | X |  |  |
| On surrounding structure / equipment | X |  |  |  |
| Below an elevated conveyor or transfer point |  | X |  |  |
| Other location (describe below) |  |  |  | X |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits |  | X |  |  |
| Localized accumulation |  | X |  |  |
| Moderate accumulation over a defined area | X |  |  |  |
| Extensive / widespread accumulation |  | X |  |  |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description | The Conveyor belt UC05, 04, 03, 02, 01 all form one logical continuous belt system which is simply broken up into physical segments to distribute the load across the distance from the crusher to the final discharge/transfer point over to the Secondary Plant(U3 Conveyor). As Such the observations where combined into one section. |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) | Photos References are: UGUC05_04_03_02_01-REF-001 through UGUC05_04_03_02_01-REF-016 |
| Other relevant observation | It was indicated that section between UC05 and UC03 was recently cleared prior to when the inspection was conducted at the time of the inspection the area is shown in UGUC05_04_03_02_01-REF-007, however these sections exhibit the same fines build up as shown in UGUC05_04_03_02_01-REF-01. |

### Central Block — Section Summary

| Common spillage pattern(s) | Dry fines spillage |
| --- | --- |
| Assets with the most significant visible accumulation | even distributed across the length of the conveyor belts sections. |
| Any spillage that appears common to several assets | Dry fines spillage |
| General comments / uncertainties |  |

## 5.2 Secondary Plant

### U3 Conveyor -Secondary Plant

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed | X |  |  |  |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  | X |  |  |
| Water carrying fines / wet fines / slurry |  | X |  |  |
| Dry fines spillage | X |  |  |  |
| Fines deposited from belt carryback on the return side | X |  |  |  |
| Lump ore / solid-product spillage |  | X |  |  |
| Mixed combined-product spillage (lump ore + fines) | X |  |  |  |
| Product intentionally cleared from a loaded belt following a trip / restart event |  |  | X |  |
| Other material or spillage type (describe below) |  |  |  | X |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point | X |  |  |  |
| Along the carrying side of the conveyor belt |  | X |  |  |
| Along one side of the belt / conveyor structure | X |  |  |  |
| Discharge / transfer point |  | X |  |  |
| Chute / screen / transfer enclosure | X |  |  |  |
| Return side / underside of the conveyor belt | X |  |  |  |
| Directly beneath the conveyor | X |  |  |  |
| Around rollers / idlers / pulleys / other conveyor components |  | X |  |  |
| Walkway / access route | X |  |  |  |
| On surrounding structure / equipment | X |  |  |  |
| Below an elevated conveyor or transfer point |  |  |  | X |
| Other location (describe below) |  |  |  | X |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits |  | X |  |  |
| Localized accumulation | X |  |  |  |
| Moderate accumulation over a defined area | X |  |  |  |
| Extensive / widespread accumulation |  | X |  |  |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) | Photo References Are: UGU3-REF-001 Though UGU3-REF-008 |
| Other relevant observation |  |

### U4 Conveyor — Tripper Car

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed | X |  |  |  |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  | X |  |  |
| Water carrying fines / wet fines / slurry |  | X |  |  |
| Dry fines spillage | X |  |  |  |
| Fines deposited from belt carryback on the return side | X |  |  |  |
| Lump ore / solid-product spillage | X |  |  |  |
| Mixed combined-product spillage (lump ore + fines) | X |  |  |  |
| Product intentionally cleared from a loaded belt following a trip / restart event |  | X |  |  |
| Other material or spillage type (describe below) |  |  |  | X |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point |  | X |  |  |
| Along the carrying side of the conveyor belt |  | X |  |  |
| Along one side of the belt / conveyor structure | X |  |  |  |
| Discharge / transfer point |  | X |  |  |
| Chute / screen / transfer enclosure | X |  |  |  |
| Return side / underside of the conveyor belt |  | X |  |  |
| Directly beneath the conveyor |  | X |  |  |
| Around rollers / idlers / pulleys / other conveyor components |  | X |  |  |
| Walkway / access route |  | X |  |  |
| On surrounding structure / equipment | X |  |  |  |
| Below an elevated conveyor or transfer point |  | X |  |  |
| Other location (describe below) |  |  |  | X |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits |  | X |  |  |
| Localized accumulation | X |  |  |  |
| Moderate accumulation over a defined area |  | X |  |  |
| Extensive / widespread accumulation | X |  |  |  |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) | Photo References are: UGU4-REF-001 Thought UGU4-REF-009 Video Refences are: UGU4-REF-010 |
| Other relevant observation | the tail end pully is misaligned as shown in photo UGU4-REF-008 and video UGU4-REF-010. This leads to Belt Mistracking as shown in photo UGU4-REF-004 and UGU4-REF-009. When the tripper car is further down the track this leads to product falling of the belt on the one side as shown in UGU4-REF-007. |

### Secondary Plant — Section Summary

| Common spillage pattern(s) | Dry fines spillage. |
| --- | --- |
| Assets with the most significant visible accumulation | U3 |
| Any spillage that appears common to several assets | Dry fines spillage, Lump ore / solid-product spillage |
| General comments / uncertainties |  |

## 5.3 Silos / Feeders

### U5A Conveyor

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed | X |  |  |  |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  | X |  |  |
| Water carrying fines / wet fines / slurry |  | X |  |  |
| Dry fines spillage | X |  |  |  |
| Fines deposited from belt carryback on the return side | X |  |  |  |
| Lump ore / solid-product spillage |  | X |  |  |
| Mixed combined-product spillage (lump ore + fines) |  | X |  |  |
| Product intentionally cleared from a loaded belt following a trip / restart event |  | X |  |  |
| Other material or spillage type (describe below) |  |  |  | X |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point | X |  |  |  |
| Along the carrying side of the conveyor belt |  | X |  |  |
| Along one side of the belt / conveyor structure |  | X |  |  |
| Discharge / transfer point |  | X |  |  |
| Chute / screen / transfer enclosure | X |  |  |  |
| Return side / underside of the conveyor belt | X |  |  |  |
| Directly beneath the conveyor | X |  |  |  |
| Around rollers / idlers / pulleys / other conveyor components | X |  |  |  |
| Walkway / access route |  | X |  |  |
| On surrounding structure / equipment | X |  |  |  |
| Below an elevated conveyor or transfer point |  | X |  |  |
| Other location (describe below) |  |  |  | X |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits |  | X |  |  |
| Localized accumulation |  | X |  |  |
| Moderate accumulation over a defined area |  |  |  |  |
| Extensive / widespread accumulation | X |  |  |  |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) | Photo References are: UG5A-REF-001 Through UG5A-REF-009 |
| Other relevant observation |  |

### U5B Conveyor

A. Was any spillage observed at or directly associated with this item?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Any spillage observed | X |  |  |  |

B. Type(s) of spillage observed

Mark all applicable rows. More than one spillage type may be present at the same location.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Water-related spillage / leakage |  | X |  |  |
| Water carrying fines / wet fines / slurry |  | X |  |  |
| Dry fines spillage | X |  |  |  |
| Fines deposited from belt carryback on the return side | X |  |  |  |
| Lump ore / solid-product spillage |  | X |  |  |
| Mixed combined-product spillage (lump ore + fines) |  | X |  |  |
| Product intentionally cleared from a loaded belt following a trip / restart event |  | X |  |  |
| Other material or spillage type (describe below) |  |  |  | X |

C. Where was the spillage physically observed?

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Loading / feed point |  | X |  |  |
| Along the carrying side of the conveyor belt |  | X |  |  |
| Along one side of the belt / conveyor structure |  | X |  |  |
| Discharge / transfer point | X |  |  |  |
| Chute / screen / transfer enclosure |  | X |  |  |
| Return side / underside of the conveyor belt | X |  |  |  |
| Directly beneath the conveyor | X |  |  |  |
| Around rollers / idlers / pulleys / other conveyor components | X |  |  |  |
| Walkway / access route |  | X |  |  |
| On surrounding structure / equipment |  | X |  |  |
| Below an elevated conveyor or transfer point | X |  |  |  |
| Other location (describe below) |  |  |  | X |

D. General visible extent

This is a qualitative indication only; no mass or volume estimate is required at this stage.

| Item / Observation | Yes | No | Unknown | N/A |
| --- | --- | --- | --- | --- |
| Trace / minor isolated deposits | X |  |  |  |
| Localized accumulation | X |  |  |  |
| Moderate accumulation over a defined area |  | X |  |  |
| Extensive / widespread accumulation |  | X |  |  |
| Unable to assess from the inspection |  |  |  | X |

E. Supporting notes / evidence

| Brief description |  |
| --- | --- |
| Approximate location / reference point |  |
| Photo / video reference(s) | Photo References are: UG5B-REF-001 Through UG5B-REF-006 |
| Other relevant observation |  |

### Silos / Feeders — Section Summary

| Common spillage pattern(s) | Dry fines spillage |
| --- | --- |
| Assets with the most significant visible accumulation | 5A |
| Any spillage that appears common to several assets | Dry fines spillage |
| General comments / uncertainties |  |

# 6. Overall Spillage Identification Summary

Complete this section only after working through the individual assets. It is intended to capture the first high-level patterns that should be folded back into the contextual notes.

| Where is water-related spillage concentrated? | Screening Plant, and CV9 that transports the wet fines from the screening plant. |
| --- | --- |
| Where are dry fines most frequently observed? | The Dry Fines are a general problem that is identified across the entire product conveyor transport eco system, it is not properly contain and transfer cleanly from one section to the next along with the solid product. |
| Where are wet fines / slurry most frequently observed? | CV9 Surface between the screens and the CV9 conveyor that transports the fines that was separated from the product. Thou instance have been partially observed in isolated instances in many other areas, as water is a common by-product that is mixed/or present in the combined product as it is transported through the conveyor system, meaning the fines is not always dry when being transported it is sometimes wet in transit. |
| Where are carryback deposits most evident? | Throughout the product conveyor transport eco system. From initial starting point all the way through to the screen plant where the fines are suppose to be collected and processed and handled from there. |
| Where is lump ore / solid-product spillage most evident? | There are some small isolated/contained area under-grounds but the most prevalent area is on the surface plant side at he loadout station conveyor system(universal across the loadout station conveyors, such as L1 etc..). |
| Where is mixed combined-product spillage most evident? | in the surface plant areas. |
| Where is trip / restart clearing material commonly deposited? | this type does not have a specific area identified as by it’s nature this typically only real happens when certain conditions happen like say when a chute get blocked for example, or the belt trips for what ever reason. For example, let’s say the belts trips and is fully loaded, if the belt struggles to startup again cause it is possibly to heavily loaded with material/product. It would be required to lighten the load by physically removing product from the belt and trip it along side the belt. This would then later require rehandling of the product dis charged from the belt as it would have to be reintroduced onto the belt so that the spillage is cleared from the site it was tipped/spilled to lighten the load. |
| Which areas show extensive or widespread accumulation? | on the surface is the CV9, and the load station sections. And the W10, |
| Any recurring pattern across several conveyors / sections? | the scrappers that are supposed to retain/ensure that the fines that sticks/retain on the return belt at the head discharge section is in-adequate by design, or in some instances not well maintained. So, fines that travels back(stuck) on the return side/return belt, of the conveyor. Elevatable get spread around/re distributed as spillage typically under the belts sections/structures or spread across other environments depending on the path that the conveyor might follow. |
| Important uncertainties requiring follow-up | None. |

# 7. Suggested Next Questionnaire Iteration

After this response is completed and incorporated into the contextual notes, the next questionnaire can focus on the cause and mechanism of each confirmed spillage condition. That iteration can test items such as belt mistracking, carryback and belt-cleaner performance, chute or skirting containment, transfer trajectory, blockages/overloading, structural condition, conveyor trips, maintenance condition, and operating / housekeeping practices.