# 1. Goals and Objectives

The primary goals of the audit are to:

- Identify the causes and contributing factors associated with combined-product spillage within the specified areas of the product transport and processing system.

- Identify the locations and operating conditions under which spillage occurs.

- Determine whether individual spillage areas are associated with a single cause or with a combination of contributing factors.

- Define practical remedial actions, engineering controls, maintenance requirements, operating procedures, or policies that may be implemented to eliminate, reduce, or control the identified spillage.

# 2. Terminology

## 2.1 Combined Product

A mixture of lump ore and fines conveyed together as a single material stream.

## 2.2 Fines

Small or finely divided ore particles forming the finer fraction of the conveyed material. This may include very fine particulate material or ore dust.

## 2.3 Belt Tracking

The lateral alignment of the conveyor belt along its intended travel path.

Correct belt tracking occurs when the belt remains centred across the conveyor structure and does not exhibit excessive lateral drift toward either side.

- Correct Belt Tracking: The belt remains centred along its intended travel path.

- Belt Mistracking: The belt moves or runs laterally away from its intended centred position.

- Tracking Deviation: The measurable lateral displacement of the conveyor belt from its intended centreline.

# 3. Report Philosophy and Assessment Approach

The purpose of the report is to document and communicate the findings of an audit conducted across selected sections of the combined-product handling and processing plant.

The audit is primarily intended to establish:

- where spillage occurs;

- what type of spillage is present;

- the mechanism or condition causing the spillage;

- whether the condition results from a single cause or a combination of contributing factors; and

- what practical actions can be taken to eliminate, reduce, contain, or otherwise control the spillage.

The report is not intended to be a fully quantified technical study of total product losses. For example, it is not necessary at this stage to determine the exact mass or volume of material spilled from every conveyor, transfer point, or processing subsystem.

The assessment therefore places greater emphasis on identifying spillage mechanisms and contributing conditions than on calculating the absolute quantity of material lost.

The extent of visible spillage may, however, be used as an indicator of the severity or persistence of a problem. For example, extensive accumulation within a particular area may indicate:

- a recurring operating condition;

- inadequate containment or cleaning;

- ineffective maintenance;

- a significant individual defect; or

- several contributing factors acting together.

The observed extent and distribution of the spillage should therefore assist in determining the probable causes and in prioritising suitable remedial actions.

Remedial actions may include, depending on the condition identified:

- mechanical modifications;

- conveyor alignment or tracking correction;

- improved product containment;

- improved fines management;

- maintenance interventions;

- structural repairs or modifications;

- improved housekeeping and spillage-removal practices;

- operational changes;

- procedures or work instructions; and

- preventative or monitoring measures.

# 4. Inspection Scope

The audit included selected sections of both the surface and underground combined-product handling and processing systems.

## 4.1 Surface Processing Plant

### 4.1.1 Screening Processing Section

- W10 Conveyor — Primary Feed

- CV01 through to CV14

### 4.1.2 Loading Station Processing Section

- L1 Conveyor — Primary Feed

- L2 Conveyor

- L3 Conveyor

- L4 Conveyor

- Sample Conveyor 1

- Sample Conveyor 2

- Sample Conveyor 3

## 4.2 Underground Sections

### 4.2.1 Central Block

- Crusher

- UC05

- UC04

- UC03

- UC02

- UC01

### 4.2.2 Secondary Plant

- U3 Conveyor

- U3 Conveyor — Tripper Car

### 4.2.3 Silos / Feeders

- U5A Conveyor

- U5B Conveyor

# 5. General Spillage Categories

The spillage identified during the audit can initially be grouped into three broad categories:

- Water-related spillage or leakage

- Fines spillage

- Solid-product / combined-product spillage

Although these categories are considered separately below, they may interact. For example, water leakage may transport fines into surrounding areas, while poor belt tracking may result in both lump ore and fines being discharged from the side of a conveyor.

# 6. Water-Related Spillage

## 6.1 Process Context

Water-related spillage occurs primarily within the screening plant, where water is introduced into the combined-product stream as part of the washing and screening process.

The water assists in separating the fines from the larger ore fraction.

## 6.2 Spillage Mechanism

Spillage occurs where the process water is not fully contained within the chute, screening, or associated process equipment.

The water may consequently leak, splash, overflow, or escape from the intended process path into the surrounding plant area.

Depending on the location and operating condition, the escaping water may also carry fines or other particulate material with it.

## 6.3 Primary Location

This form of spillage appears to occur primarily within the Screening Processing Section.

## 6.4 Potential Consequences

Water escaping from the process may contribute to:

- wet or slippery walking surfaces;

- transport of fines into surrounding areas;

- accumulation of wet fines or slurry;

- accelerated corrosion or degradation of surrounding equipment and structures;

- increased housekeeping requirements; and

- reduced effectiveness of general spillage-control measures.

# 7. Fines Spillage

## 7.1 Generation and Process Context

Fines are present throughout much of the combined-product handling process.

A significant proportion of the fines may be generated during crushing and subsequent material handling.

The fines are then transported together with the larger ore fraction through the conveyor system until the washing and screening process separates the finer material from the larger product.

## 7.2 Primary Fines-Spillage Mechanism — Carryback

A major fines-spillage mechanism occurs when fines remain adhered to the conveyor belt after the main product has been discharged at a transfer or discharge point.

Instead of leaving the belt with the bulk product, a portion of the fines remains attached to the belt surface and travels onto the return side of the conveyor.

This condition may be referred to as belt carryback.

As the return belt travels back through the conveyor system, the adhered fines may progressively detach from the belt.

The fines can then:

- fall from the underside of the return belt;

- be deposited below the conveyor;

- accumulate around conveyor structures;

- fall into walkways or access areas; or

- be dislodged when the belt comes into contact with mechanical components such as return rollers and other conveyor-support components.

## 7.3 Distribution

Because carryback occurs on the return side of the conveyor, fines spillage can potentially extend for a considerable distance downstream of the original discharge point.

This means the physical location where fines accumulate is not necessarily the location where the underlying cause originates.

Fines spillage may therefore potentially occur throughout significant portions of the conveyor system.

## 7.4 When Fines Become a Spillage Problem

Fines become problematic where they accumulate outside the intended product-handling path, particularly where they:

- accumulate within walkways or access areas;

- collect beneath conveyors;

- build up around rollers or other conveyor components;

- interfere with equipment operation or maintenance access;

- create excessive housekeeping requirements;

- become wet and form mud or slurry;

- migrate into surrounding equipment or structures; or

- create an unsafe working environment.

# 8. Solid Product / Combined-Product Spillage

Solid-product spillage refers primarily to lump ore or the combined product physically leaving the intended conveyor or transfer path.

Several different mechanisms may result in this form of spillage.

## 8.1 Belt Mistracking

When conveyor belt mistracking causes the belt to move laterally away from its intended centred position, the effective conveying area becomes displaced relative to the intended product path.

This may create an exposed area or gap along one side of the conveyor, allowing ore or combined product to spill from the side of the belt.

The severity of the resulting spillage may depend on factors such as:

- the magnitude of the tracking deviation;

- the quantity of material being conveyed;

- material loading distribution;

- conveyor speed;

- belt trough profile;

- containment arrangements; and

- the position of the mistracking relative to transfer points or other equipment.

## 8.2 Conveyor Trips and Restart Preparation

When a loaded conveyor trips, it may be necessary to remove a portion of the conveyed product from the belt in order to reduce the static load and facilitate conveyor restart.

If the removed material is deposited adjacent to the conveyor or within surrounding plant areas without an effective recovery or containment method, this material becomes a further source of product spillage.

The resulting spillage is therefore not necessarily caused by a conveyor defect alone, but may arise as a secondary consequence of the trip and subsequent restart procedure.

## 8.3 Transfer and Containment Losses

Solid product may also escape where the material is transferred between conveyor belts or through chutes if the product trajectory is not adequately controlled or contained.

Potential contributing factors may include:

- inadequate chute containment;

- worn or damaged chute components;

- incorrect material trajectory;

- excessive material loading;

- belt mistracking at the transfer point;

- blockages or restrictions;

- poor skirt or sealing arrangements; or

- structural or mechanical deterioration.

These mechanisms should be assessed individually at each identified spillage location.

## 8.4 When Solid-Product Spillage Becomes a Problem

Solid-product spillage becomes particularly significant when material:

- enters walkways or access routes;

- accumulates around operating equipment;

- interferes with plant operation or maintenance;

- obstructs access to equipment;

- falls from elevated conveyors;

- creates additional manual handling requirements; or

- creates the potential for injury or damage to personnel, structures, or equipment below.

Material falling from an elevated conveyor may represent a greater consequence than equivalent spillage occurring at ground level because of the additional impact energy associated with the falling product.

## 8.5 Potential Locations

Solid-product spillage may occur throughout the conveyor system, depending on the particular mechanical, operational, transfer, or containment condition present at each location.

# 9. Relationship Between Spillage Location and Root Cause

An important principle for the audit is that the location where spilled material is found may not necessarily be the location where the original problem occurs.

Examples include:

- fines adhering to a belt at a discharge point and only falling from the belt further along the return path;

- belt mistracking developing upstream and causing product loss at a downstream containment point;

- product removed following a conveyor trip accumulating some distance from the point that caused the trip; and

- water carrying fines away from the original leakage point.

The investigation should therefore distinguish between:

- the location where the spillage is observed;

- the mechanism by which the material reached that location; and

- the original condition or root cause responsible for the spillage.

This distinction will be important when determining suitable corrective actions.

# 10. Initial Problem Areas / Restrictions Identified

The audit has identified several broader problem areas that may influence the occurrence, severity, or management of spillage.

These should be developed further as the individual findings are assessed.

## 10.1 Conveyor Mechanical Component Maintenance

The condition, adjustment, wear, and maintenance of conveyor components may influence:

- belt tracking;

- belt cleaning;

- material containment;

- product transfer;

- carryback; and

- general conveyor reliability.

Relevant components may include rollers, idlers, pulleys, belt cleaners, skirting, chute components, tensioning systems, and associated alignment components.

## 10.2 Structural Maintenance

Structural deterioration, deformation, movement, or misalignment may affect conveyor geometry and the performance of associated containment systems.

Structural maintenance may therefore have a direct or indirect influence on spillage.

## 10.3 Confined-Space and Restricted-Access Areas

Some spillage locations may be difficult to inspect, clean, maintain, or modify because of confined-space requirements or restricted physical access.

These restrictions may influence both:

- the persistence of existing spillage; and

- the practicality of implementing corrective actions.

## 10.4 Fines Control

Fines require specific consideration because their behaviour differs from that of larger lump ore.

Control measures may need to address:

- carryback;

- belt cleaning;

- dust;

- wet fines or slurry;

- accumulation beneath conveyors; and

- migration into surrounding plant areas.

## 10.5 Spillage Handling

The method by which spilled product is collected, removed, stored, and returned to the process should be assessed.

Spillage handling should not only address the removal of material after an event but should also consider how recovery activities themselves may contribute to secondary spillage.

## 10.6 Spillage-Control Measures

Existing preventative and containment measures should be assessed to determine whether they are:

- present;

- correctly installed;

- appropriate for the application;

- maintained;

- effective under normal operating conditions; and

- effective during abnormal operating conditions such as conveyor trips or blockages.

# 11. Initial Cause-and-Effect Framework

The individual observations within the report can be assessed using the following general sequence:

Observed Spillage → Spillage Type → Immediate Mechanism → Contributing Condition(s) → Root Cause → Consequence → Existing Control → Recommended Remedial Action

For example:

Product accumulation below conveyor
→ Fines
→ Carryback from return belt
→ Fines adhering to belt after discharge
→ Ineffective or inadequate belt cleaning
→ Fines deposited along conveyor return path
→ Existing cleaner / housekeeping practices assessed
→ Improve belt-cleaning effectiveness and verify cleaner configuration.

Or:

Lump ore adjacent to conveyor
→ Solid-product spillage
→ Product leaves side of belt
→ Belt operating laterally offset
→ Belt mistracking / conveyor alignment issue
→ Product loss and potential personnel/equipment hazard
→ Existing tracking arrangements assessed
→ Correct underlying tracking condition and verify containment.

# 12. Intended Outcome of the Audit

The final report should ultimately provide a practical understanding of:

- Where is the spillage occurring?

- What type of material is being spilled?

- How is the material escaping from the process?

- Why is it occurring?

- Is the problem isolated or recurring?

- Is one cause responsible, or are several factors interacting?

- What consequence does the condition create?

- What measures are currently in place?

- Why are those measures not fully effective?

- What corrective or preventative actions should be implemented?

The primary objective is therefore not simply to document spilled material, but to establish the relationship between the observed spillage, the operating or mechanical mechanism responsible for it, and the practical action required to control or eliminate the problem.